Configuration Guide: BPM - CG PC Measured Demand Over Control Area

Version 5.13.5

Table of Contents

1.	Purpose of Document	3

2.	Introduction	3

2.1	Background	3

2.2	Description	8

3.	Charge Code Requirements	9

3.1	Business Rules	9

3.2	Predecessor Charge Codes	14

3.3	Successor Charge Codes	14

3.4	Inputs – External Systems	15

3.5	Inputs – Predecessor Charge Codes or Pre-calculations	20

3.6	CAISO Formula	23

3.7	Outputs	72

4.	Charge Code Effective Date	103

# Purpose of Document

The purpose of this document is to capture the requirements and design specification for a Charge Code in one document.

# Introduction

## Background

In the allocation of various payment and charges, the amounts are divided among Business Associates (BAs) in proportion to their relative contribution to the overall Measured Demand quantity (in MWh) over which the allocation is performed.  Thus, an allocation of payment or charges requires the following two Measured Demand calculations that are conducted for the particular time period over which the allocation is performed:

- the overall metered CAISO Demand plus Real-Time Interchange export schedules, excluding that portion of Demand of Non-Generator Resources dispatched as Regulation through Regulation Energy Management, in the grid sector or area over which the allocated payments and charges apply and

- the contribution of each Business Associate to the overall CAISO Demand plus Real-Time Interchange export schedules, excluding that portion of Demand of Non-Generator Resources dispatched as Regulation through Regulation Energy Management.

To provide the Measured Demand quantities that are used in multiple charge code calculations that allocate various payments or charges, the CAISO implements a set of Pre-calculation processes, referred to as Measured Demand Pre-calculations, to calculate particular Measured Demand quantities.  There are five (5) Measured Demand Pre-calculation configurations in which an overall Measured Demand quantity is calculated.  The five Pre-calculations differ by the composition of the Demand resources, and exports that are included in their Measured Demand outputs.  The Measured Demand Pre-calculation process configurations, the Demand resources and export composition of their outputs, and the associated output frequency are identified as follows:

| Measured Demand Pre-calculation Configurations |  |  |
| --- | --- | --- |
| Name | Output Composition and Frequency | Dependent Allocation Configurations |
| MD over Control Area Pre-calculation | Composition: Measured Demand: The sum (as a negative value) of all - metered CAISO Demand* - Real-Time Interchange export schedules Net MSS Measured Demand: The lesser of - zero (0) or the net value defined as - the sum of Meter Data (as a negative value) for Demand of all Load and other non-export Demand resources internal to an MSS plus (+) the sum of - Meter Data representing Generation of all Generating Units for the MSS to which lesser value is added - the sum of all of the MSS Real-Time Interchange export schedules (as a negative value). Frequency: 5-minute,10-minute, hourly, daily | CC 6474 – Real Time Unaccounted for Energy Settlement |
|  |  | CC 6475 – Real Time Uninstructed Imbalance Energy Settlement |
|  |  | CC 6477 – Real Time Imbalance Energy Offset |
|  |  | CC 6480 – Excess Cost Neutrality Allocation |
|  |  | CC 6636 – IFM Bid Cost Recovery Tier 1 Allocation |
|  |  | CC 6637 – IFM Bid Cost Recovery Tier 2 Allocation |
|  |  | CC 6678 – Real Time Bid Cost Recovery Allocation |
|  |  | CC 6774 – RT Congestion Offset |
|  |  | CC 6790 – CRR Balancing Account |
|  |  | CC 6806 – Day Ahead Residual Unit Commitment (RUC) Tier 1 Allocation |
|  |  | CC 6807 – Day Ahead Residual Unit Commitment (RUC) Tier 2 Allocation |
|  |  | CC 6947 – IFM Marginal Losses Surplus Credit Allocation |
|  |  | CC 550 – Monthly FERC Fee Settlement |
|  |  | CC 551 – Annual FERC Fee Settlement |
|  |  | CC 4989 – Daily Rounding Adjustment Allocation |
|  |  | CC 4999 – Monthly Rounding Adjustment |
|  |  | CC 6457 – Declined Hourly Pre-Dispatch Penalty Allocation |
|  |  | CC 7056 – Flexible Ramp Cost Allocation |
|  |  | CC 8999 – Neutrality Adjustment |
|  |  | PC – Metered Demand Over TAC Area And CPM |
|  |  | PC – Standard Capacity Product |
| MD over Control Area Excl MSS Energy Pre-calculation | Composition: Measured Demand: The sum (as a negative value) of all - metered CAISO Demand - Real-Time Interchange export schedules The overall Measured Demand output quantity excludes Energy from MSS entities. Frequency: 5-minute,10-minute and monthly | CC 1302 – Long-Term Voltage Support Allocation |
|  |  | CC 1303 – Supplemental Reactive Energy Allocation |
| MD Emissions over Control Area Excl External Exports Pre-calculation | Composition: Measured Demand (in-state): The sum (as a negative value) of all - metered CAISO Demand - in-state Real-Time Interchange export schedules The sum excludes CAISO-external exports and any external contractual Transmission Loss adjustments. Net MSS Measured Demand (in-state): The lesser of - zero (0) or the net value defined as - the sum of Meter Data (as a negative value) for Demand of all Load and other non-export Demand resources internal to an MSS plus (+) the sum of - Meter Data representing Generation of all Generating Units for the MSS to which lesser value is added - the sum of all of the MSS in-state Real-Time Interchange export schedules (as a negative value). The sum excludes CAISO-external exports and any external contractual Transmission Loss adjustments. Frequency: Monthly | CC 591 – Emissions Cost Recovery |
| MD Black Start Excl Exports Pre-calculation | Composition: Metered CAISO Demand: the sum of all metered CAISO Demand (as a negative value). Net MSS Demand: The greater of - zero (0) or the net value defined as - the sum of Meter Data (as a negative value) for Demand of all Load and other non-export Demand resources internal to an MSS, plus (+) the sum of - Meter Data representing Generation of all Generating Units for the MSS. Frequency: Hourly | CC 1353- Black Start Energy Allocation |
| MD over Control Area Excl Transmission Loss Adjustment Pre-calculation | Composition: Measured Demand: The sum (as a negative value) of all - metered CAISO Demand - Real-Time Interchange export schedules The calculated value excludes contractual Transmission Loss adjustments. Net MSS Measured Demand: The greater of - zero (0) or the net value defined as - the sum of Meter Data for Demand (as a negative value) of all Load and other non-export Demand resources internal to an MSS plus (+) the sum of - Meter Data representing Generation of all Generating Units for the MSS to which lesser value is added - the sum of all of the MSS Real-Time Interchange export schedules (as a negative value). The calculated value excludes contractual Transmission Loss adjustments. Frequency: 5-minute,10-minute and Hourly | CC 6977 – Allocation of Transmission Loss Obligation Charge for Real Time Schedules under Control Agreement |

Note:

* 	Metered CAISO Demand excludes that portion of Demand of Non-Generator Resources dispatched as Regulation through Regulation Energy Management.

Some of the Measured Demand configurations listed above recognize exemptions to the calculation for various Business Associates and resources.  In some instances a Measured Demand configuration may produce multiple outputs, each one recognizing a particular exemption or set of exemptions.  For each Measured Demand calculation, its exemptions are determined and defined by the requirements of the allocations in which the Measured Demand Pre-calculation outputs are employed.

All of the dependent allocation charge codes listed in the above Measured Demand Pre-calculation configuration summary table do not necessarily use the entire set of Measured Demand output components that the table associates with the charge codes. Some of the outputs used in the allocations are adjusted for exceptions as explained in the prior paragraph. Moreover some outputs used in various allocations are adjusted for various transmission-related contracts in order to exclude Measured Demand that is associated with contract rights from being used in a calculation. Additionally, based on the MRTU Tariff, special rules apply for the application of Measured Demand components for MSS entities that Load Follow, or for the allocation of Unaccounted-For-Energy (UFE). Some allocations may be based on metered CAISO Demand, not the full Measured Demand quantity. Thus, the configuration summary table above is quite simplistic and is intended to present only a top-level view of the output components of the Measured Demand Pre-calculations.

## Description

Generically, the Measured Demand calculations represent the sum of metered CAISO Demand (and net MSS Demand for net-settled MSS entities) and Real-Time Interchange export schedules, excluding that portion of Demand of Non-Generator Resources dispatched as Regulation through Regulation Energy Management.  Net MSS Demand is defined to be the greater of zero (0) or the net of Meter Data for the Demand of all Load and other non-export Demand resources internal to an MSS, with the Meter Data representing the Generation of all MSS Generating Units.  Depending upon the exact Measured Demand calculation, various export components are included or excluded in a summed Measured Demand output value.  In the “Measured Demand over Control Area Excl Transmission Loss Adjustment” output calculations, contractual Transmission Losses are excluded as exports (where contractual Transmission Losses are defined to be operational adjustments that are scheduled as export Energy according to operating agreements between transmission line operators and the CAISO).  In Measured Demand calculations other than those of the “Measured Demand over Control Area Excl Transmission Loss Adjustment” process, the contractual Transmission Loss export Energy is included.  In addition to contractual Transmission Losses, the various Measured Demand Pre-calculations vary in the inclusion of out-of-state exports, or whether any exports at all are included in Measured Demand outputs.  The “BPM - CG PC Measured Demand Over Control Area” outputs defined in the current document include all exports, both in-state and out-of-state, as well as contractual Transmission Losses.

The “BPM - CG PC Measured Demand Over Control Area” process derives its Measured Demand quantities from all metered CAISO Demand and scheduled Real-Time Interchange export quantities in the CAISO Control Area, excluding that portion of Demand of Non-Generator Resources dispatched as Regulation through Regulation Energy Management.  Providing for the needs of multiple successor processes, the Pre-calculation calculates a set of Measured Demand outputs. Measured Demand quantities are output at various frequencies. The outputs are expected to serve the allocation charge codes, as stipulated in the prior section. The calculation produces an output at multiple levels of summation for use in allocating charges and payments. For example, the Measured Demand output appears as the sum of the metered CAISO Demand and scheduled Real-Time Interchange export quantities, excluding that portion of Demand of Non-Generator Resources dispatched as Regulation through Regulation Energy Management, for each Business Associate of the CAISO Control Area. The Measured Demand quantity is then summed over all Business Associates to formulate a value for the overall CAISO Control Area.

# Charge Code Requirements

## Business Rules

| Bus Req ID | Business Rule |
| --- | --- |
|  | The BPM - CG PC Measured Demand Over Control Area shall perform a set of calculations to determine and output Measured Demand (metered CAISO Demand plus Real-Time Interchange export schedules) over the CAISO Control Area, excluding that portion of Demand of Non-Generator Resources dispatched as Regulation through Regulation Energy Management. |
|  | Measured Demand shall be calculated and output as a total quantity for the overall CAISO Control Area. |
|  | Measured Demand shall be calculated and output as a total quantity for each Utility Distribution Company (UDC) over the CAISO Control Area. |
|  | Measured Demand shall be calculated and output as a total quantity over the CAISO Control Area for each Business Associate. |
|  | Measured Demand shall be calculated and output as a total quantity for each combination of UDC and Business Associate over the CAISO Control Area. |
|  | Contractual Transmission Losses, representing Transmission Losses that are covered by agreements between transmission line operators and the CAISO, shall be included in a Measured Demand calculation as Real-Time Interchange export schedules. |
|  | The Measured Demand over Control Area Pre-calculation shall provide two types of Measured Demand outputs for Metered Subsystem (MSS) entities as stipulated in Business Rules 2.1 and 2.2 below. |
|  | For Metered Subsystem (MSS) entities for which Energy is settled via the net-settlement option or where the MSS has selected Load-Following, the Measured Demand over Control Area Pre-calculation shall provide outputs that present MSS Measured Demand based on net MSS Measured Demand. |
|  | The net MSS Measured Demand quantity shall be input from the MSS Netting Pre-calculation process. |
|  | For Metered Subsystem (MSS) entities for which Energy is settled via the gross-settlement option (regardless of whether or not the Load-Following option has been selected), the Measured Demand over Control Area Pre-calculation shall provide outputs that present MSS Measured Demand based on Metered Quantities for MSS Load Demand, excluding that portion of Demand of Non-Generator Resources dispatched as Regulation through Regulation Energy Management, plus Real-Time Interchange export schedules from the MSS. |
|  | For Load-Following Metered Subsystem (MSS) entities (that have selected the MSS Load-Following option) for which Energy delivered to the CAISO is settled via the gross-settlement option, the Measured Demand over Control Area Pre-calculation shall provide 2 sets of outputs, one set that presents MSS Measured Demand based on Metered Quantities for MSS Load Demand, excluding that portion of Demand of Non-Generator Resources dispatched as Regulation through Regulation Energy Management, plus Real-Time Interchange export schedules from the MSS (per business rule 2.2), and the other set that presents MSS Measured Demand based on net MSS Measured Demand (per business rule 2.1). |
|  | For MSS entites, regardless of settlement selection options, the BPM - CG PC Measured Demand Over Control Area Pre-calculation shall provide a set of MSS Measured Demand outputs based on net Measured Demand. |
|  | The BPM - CG PC Measured Demand Over Control Area shall provide Measured Demand outputs that exclude particular combinations of Business Associate (BA) and resource values. |
|  | Exceptions shall be defined in a standing data table. |
|  | Exceptions shall be organized in the standing data table as “exception sets”. |
|  | An exception set shall be used to calculate at most one set of Measured Demand outputs. |
|  | Multiple exception sets may be employed to calculate a set of Measured Demand outputs. |
|  | Exception flag inputs shall be provided for each exception set in order to indicate when an exception condition within the exception set is valid. |
|  | The current implementation of the BPM - CG PC Measured Demand Over Control Area shall provide for nine (9) exception sets, referred to as “Exceptions #1” through “Exceptions #9”. |
|  | Each exception set shall be effective-dated (with a start data and end date) to indicate the period over which the exception set is in effect. |
|  | The Measured Demand over Control Area Pre-calculation shall compute Measured Demand outputs that subtract Generation and Transmission Rights from the Measured Demand quantity modified for Exceptions, for a set of generators that allow for an offset against Measured Demand for a FERC Fee allocation and for a set of BA-associated transmission rights. |
|  | The BPM - CG PC Measured Demand Over Control Area shall provide Measured Demand outputs that exclude Measured Demand associated with transmission rights (in MWh) associated with particular Business Associate (BA) values. |
|  | Measured Demand Outputs shall be provided that exclude all Measured Demand associated with transmission rights for a particular Business Associate (BA) value. |
|  | The Measured Demand Over Control Area Pre-calculation shall provide Measured Demand Outputs that exclude all Measured Demand associated with real-time ETC and TOR transmission rights for a particular Business Associate (BA) value. |
|  | The Measured Demand Over Control Area Pre-calculation shall provide Measured Demand Outputs that exclude all Measured Demand associated with real-time TOR transmission rights for a particular Business Associate (BA) value. |
|  | The Measured Demand Over Control Area Pre-calculation shall output the Metered CAISO Demand quantity by Aggregated Price Node. |
|  | The Metered CAISO Demand quantity shall consist of the sum of (a) the interval metered CAISO Demand quantity over all UDC Load and Non-Generator Resources (NGR), excluding that portion of Demand of Non-Generator Resources dispatched as Regulation through Regulation Energy Management, and gross-settled MSS Demand (over all MSS Load and NGR resources, excluding that portion of Demand of Non-Generator Resources dispatched as Regulation through Regulation Energy Management), where the Energy is associated with a default LAP and (b) the interval Net MSS Demand quantity over all net-settled MSS entities, where the Energy is associated with a MSS/custom LAP. |
|  | The Metered CAISO Demand quantity output shall be calculated on both a Settlement Interval and an hourly basis. In addition, selected outputs shall be calculated on a 10-minute interval basis. |
|  | A Metered CAISO Demand quantity output shall be calculated for each Business Associate (BA) and, additionally, for the entire CAISO Control Area as the sum of the individual BA quantities. |
|  | The Measured Demand Over Control Area Pre-calculation shall, in addition to the outputs that represent the Tariff-defined Measured Demand and its components (Metered CAISO Demand, excluding that portion of Demand of Non-Generator Resources dispatched as Regulation through Regulation Energy Management, and Real-Time Interchange Export Schedules), provide a similar set of outputs (for gross Measured Demand) that represent MSS Demand on a gross-settled basis, regardless of MSS Gross/Net Energy settlement type. |
|  | The Measured Demand output shall include Metered CAISO Demand, excluding that portion of Demand of Non-Generator Resources dispatched as Regulation through Regulation Energy Management, plus Real-Time Interchange Export Schedules for all resources associated with a UDC or a gross-settled MSS entity. |
|  | The Measured Demand output shall include MSS gross Demand, excluding that portion of Demand of Non-Generator Resources dispatched as Regulation through Regulation Energy Management, plus Real-Time Interchange Export Schedules for all net-settled MSS entities. |
|  | The Measured Demand Over Control Area Pre-calculation shall, in addition to the outputs that represent the Tariff-defined Metered CAISO Demand (excluding that portion of Demand of Non-Generator Resources dispatched as Regulation through Regulation Energy Management), provide outputs for gross Metered CAISO Demand that represent MSS Metered Demand on a gross-settled basis, regardless of MSS Gross/Net Energy settlement type. |
|  | For use in BCR Settlement, the Measured Demand Over Control Area Pre-calculation shall present a set of Metered Demand outputs by BA and CAISO Control Area. |
|  | Metered Demand outputs for BCR Settlement shall be calculated with transmission ownership rights (TORs) associated with a BA’s Loads being subtracted from the overall Metered Demand quantities for the BA. |
|  | The BCR-related Measured Demand exceptions (stipulated in the Exceptions #6 exceptions) shall be applied to the calculation of a BA’s BCR-related Metered Demand and Measured Demand outputs. |
|  | The Measured Demand Over Control Area Pre-calculation shall output for all UDC and MSS Gross Load the Metered CAISO Demand quantity, excluding that portion of Demand of Non-Generator Resources dispatched as Regulation through Regulation Energy Management, minus the valid and balanced TOR contract portion of Load Self-Schedules having TOR contract rights that are eligible for Congestion credits. |
|  | The Metered CAISO Demand minus valid and balanced TOR quantity, excluding that portion of Demand of Non-Generator Resources dispatched as Regulation through Regulation Energy Management, shall be calculated for each Transmission Access Charge (TAC) Area. |
|  | The Metered CAISO Demand minus valid and balanced TOR quantity, excluding that portion of Demand of Non-Generator Resources dispatched as Regulation through Regulation Energy Management, shall be calculated for each Gross Load within the TAC Area. |
|  | The total of Metered CAISO Demand minus valid and balanced TOR quantity, excluding that portion of Demand of Non-Generator Resources dispatched as Regulation through Regulation Energy Management, shall be calculated over the Gross Load resources scheduled within each TAC Area and shall be broken down and output by BA and TAC Area. |
|  | The TAC Area total of Metered CAISO Demand minus valid and balanced TOR quantity, excluding that portion of Demand of Non-Generator Resources dispatched as Regulation through Regulation Energy Management, shall be calculated over all BAs who schedule Gross Load within the TAC Area. |

## Predecessor Charge Codes

Charge Code/ Pre-calc Name

Allocation of Transmission Losses Under Control Agreements Pre-calculation

System Resource Deemed Delivered Energy Pre-calculation

MSS Netting Pre-calculation

ETC, TOR, CVR Quantity Pre-calculation

## Successor Charge Codes

Charge Code/ Pre-calc Name

CC 4989 – Daily Rounding Adjustment Allocation

CC 4999 – Monthly Rounding Adjustment

CC 550 – Monthly FERC Fee Settlement

CC 551 – Annual FERC Fee Settlement

CC 6457 – Declined Hourly Pre-Dispatch Penalty Allocation

CC 6474 – Real Time Unaccounted for Energy Settlement

CC 6475 – Real Time Uninstructed Imbalance Energy Settlement

CC 6477 – Real Time Imbalance Energy Offset

CC 6480 – Excess Cost Neutrality Allocation

CC 6636 – IFM Bid Cost Recovery Tier 1 Allocation

CC 6637 – IFM Bid Cost Recovery Tier 2 Allocation

CC 6678 – Real Time Bid Cost Recovery Allocation

CC 6774 – RT Congestion Offset

CC 6790 – CRR Balancing Account

CC 6806 – Day Ahead Residual (RUC) Tier 1 Allocation

CC 6807 – Day Ahead Residual (RUC) Tier 2 Allocation

CC 6947 – IFM Marginal Loss Surplus Credit Allocation

CC 6985 – Real Time Marginal Losses Offset

CC 7056 – Flexible Ramp Cost Allocation

CC 8989 – Dailly Neutrality Adjustment

CC 8999 – Monthly Neutrality Adjustment

PC – Metered Demand Over TAC Area And CPM

PC – Standard Capacity Product

PC – RealTime Congestion

## Inputs – External Systems

Attributes that are listed for a variable name in the table below, but are not described in the associated description are incidental to and do not materially participate in the calculations of Section 3.6 herein.

| Row # | Variable Name | Description |
| --- | --- | --- |
|  | MeasuredDemandControlAreaExceptions1Flag Brt | Flag value (a binary 0-1 indicator) derived by the data mapping process from the Exceptions #1 standing data input, that indicates whether a condition from Exceptions #1 is met (true) for a given combination of Business Associate, and resource ID values. When an Exceptions #1 condition is true, the BPM - CG PC Measured Demand Over Control Area shall exclude any associated metered CAISODemand, net MSS Measured Demand or export input from the calculation of the CAISOTotalHourlyMeasuredDemandControlAreaQty_Ex1hi and BAHourlyMeasuredDemandControlAreaQty_Ex1Bmdhi Measured Demand outputs. |
|  | BAMeasuredDemandCAMDExceptionsFlag B | Flag value (a binary 0-1 indicator) derived by the data mapping process from the Exceptions #1 standing data input, that indicates whether a condition from Exceptions #1 is met (true) for a given value for a Business Associate. When an Exceptions #1 condition is true, the BPM - CG PC Measured Demand Over Control Area shall exclude any associated metered CAISODemand, net MSS Measured Demand or export input from the calculation of the CAISOTotalHourlyMeasuredDemandControlAreaQty_Ex1hi and BAHourlyMeasuredDemandControlAreaQty_Ex1Bmdhi Measured Demand outputs. |
|  | MeasuredDemandControlAreaExceptions2Flag Brt | Flag value (a binary 0-1 indicator) derived by the data mapping process from the Exceptions #2 standing data input, that indicates whether a condition from Exceptions #2 is met (true) for a given combination of Business Associateand resource ID values. When an Exceptions #2 condition is true, the BPM - CG PC Measured Demand Over Control Area shall exclude any associated meter Load, NGR Demand, or export input from the calculation of the CAISOTotalDailyMeasuredDemandControlAreaQty_FERCmd and BADailyMeasuredDemandControlAreaQty_FERCBmd Measured Demand outputs. |
|  | BAMeasuredDemandFERCExceptionsFlag B | Flag value (a binary 0-1 indicator) derived by the data mapping process from the Exceptions #2 standing data input, that indicates whether a condition from Exceptions #2 is met (true) for a given value for a Business Associate. When an Exceptions #2 condition is true, the BPM - CG PC Measured Demand Over Control Area shall exclude any associated meter Load, NGR resource, or export input from the calculation of the CAISOTotalDailyMeasuredDemandControlAreaQty_FERCmd and BADailyMeasuredDemandControlAreaQty_FERCBmd Measured Demand outputs. |
|  | MeasuredDemandControlAreaExceptions3Flag Brt | Flag value, derived by the data mapping process from the Exceptions #3 standing data input, that indicates whether a condition from Exceptions #3 is met (true) for a given combination of Business Associate and resource ID values. When an Exceptions #3 condition is true, the BPM - CG PC Measured Demand Over Control Area shall include any associated metered Generation input in the calculation of the CAISOTotalDailyMeasuredDemandControlAreaQty_FERCmd and BADailyMeasuredDemandControlAreaQty_FERCBmd Measured Demand outputs. |
|  | BAMeasuredDemandGenFERCExceptionsFlag B | Flag value, derived by the data mapping process from the Exceptions #3 standing data input, that indicates whether a condition from Exceptions #3 is met (true) for a given value for a Business Associate.When an Exceptions #3 condition is true, the BPM - CG PC Measured Demand Over Control Area shall include any associated metered Generation input in the calculation of the CAISOTotalDailyMeasuredDemandControlAreaQty_FERCmd and BADailyMeasuredDemandControlAreaQty_FERCBmd Measured Demand outputs. |
|  | MeasuredDemandControlAreaExceptions5Flag B | Flag value, derived by the data mapping process from the Exceptions #5 standing data input, that indicates whether a condition from Exceptions #5 is met (true) for a given value for a Business Associate. When an Exceptions #5 condition is true, the BPM - CG PC Measured Demand Over Control Area shall exclude any associated transmission rights Energy from the calculation of various Measured Demand outputs. For outputs where another exceptions set input also applies (for instance, MeasuredDemandControlAreaExceptions1Flag Brt), the other exceptions set is presumed to exclude any B (or resource r scheduled by B) that is included in Exceptions #5. |
|  | MeasuredDemandControlAreaExceptions6Flag Brt | Flag value (a binary 0-1 indicator) derived by the data mapping process from the Exceptions #6 standing data input, that indicates whether a condition from Exceptions #6 is met (true) for a given combination of Business Associate and resource ID values. When an Exceptions #6 condition is true, the BPM - CG PC Measured Demand Over Control Area shall exclude any associated metered CAISODemand, net MSS Measured Demand or export input from the calculation of the CAISOTotalHourlyMeasuredDemandControlAreaQty_Ex6 mdhi and BAHourlyMeasuredDemandControlAreaQty_Ex6 Bmdhi Measured Demand outputs. |
|  | BAMeasuredDemandBCRExceptionsFlag B | Flag value (a binary 0-1 indicator) derived by the data mapping process from the Exceptions #6 standing data input, that indicates whether a condition from Exceptions #6 is met (true) for a given value for a Business Associate. When an Exceptions #6 condition is true, the BPM - CG PC Measured Demand Over Control Area shall exclude any associated metered CAISODemand, net MSS Measured Demand or export input from the calculation of the CAISOTotalHourlyMeasuredDemandControlAreaQty_Ex6 mdhi and BAHourlyMeasuredDemandControlAreaQty_Ex6 Bmdhi Measured Demand outputs. |
|  | MeasuredDemandControlAreaCONGOFFExceptions7Flag Brt | Flag value (a binary 0-1 indicator) derived by the data mapping process from the Exceptions #7 standing data input, that indicates whether a condition from Exceptions #7 is met (true) for a given combination of Business Associate and resource ID values. When an Exceptions #7 condition is true, the BPM - CG PC Measured Demand Over Control Area shall exclude any associated metered CAISODemand, net MSS Measured Demand or export input from the calculation of the CAISOHourlyMeasuredDemandMinusBalancedRightsQuantity_EX_RTM_CONGOFF mdh and BAHourlyMeasuredDemandMinusBalancedRightsQuantity_EX_RTM_CONGOFF Bmdh Measured Demand outputs. |
|  | BAMeasuredDemandCONGOFFExceptions7Flag B | Flag value (a binary 0-1 indicator) derived by the data mapping process from the Exceptions #7 standing data input, that indicates whether a condition from Exceptions #7 is met (true) for a given value for a Business Associate. When an Exceptions #7 condition is true, the BPM - CG PC Measured Demand Over Control Area shall exclude any associated metered CAISODemand, net MSS Measured Demand or export input from the calculation of the CAISOHourlyMeasuredDemandMinusBalancedRightsQuantity_EX_RTM_CONGOFF mdh and BAHourlyMeasuredDemandMinusBalancedRightsQuantity_EX_RTM_CONGOFF Bmdh Measured Demand outputs. |
|  | MeasuredDemandControlAreaIMBOFFExceptions8Flag Brt | Flag value (a binary 0-1 indicator) derived by the data mapping process from the Exceptions #8 standing data input, that indicates whether a condition from Exceptions #8 is met (true) for a given combination of Business Associate and resource ID values. When an Exceptions #8 condition is true, the BPM - CG PC Measured Demand Over Control Area shall exclude any associated metered CAISODemand, net MSS Measured Demand or export input from the calculation of the CAISOSettlementIntervalMeasuredDemandMinusBalancedTORLossQuantity_EX_RTM_IMBOFF mdhi, BASettlementIntervalMeasuredDemandMinusBalancedTORLossQuantity_EX_RTM_IMBOFF Bmdhi, CAISOSettlementIntervalMeasuredDemandMinusBalancedTORDemandQuantity_EX_RTM_IMBOFF mdhi and BASettlementIntervalMeasuredDemandMinusBalancedTORDemandQuantity_EX_RTM_IMBOFF Bmdhi Measured Demand outputs. |
|  | BAMeasuredDemandIMBOFFExceptions8Flag B | Flag value (a binary 0-1 indicator) derived by the data mapping process from the Exceptions #8 standing data input, that indicates whether a condition from Exceptions #8 is met (true) for a given value for a Business Associate. When an Exceptions #8 condition is true, the BPM - CG PC Measured Demand Over Control Area shall exclude any associated metered CAISODemand, net MSS Measured Demand or export input from the calculation of the CAISOSettlementIntervalMeasuredDemandMinusTORQuantity_EX_RTM_IMBOFF mdhi and BASettlementIntervalMeasuredDemandMinusTORQuantity_EX_RTM_IMBOFF Bmdhi Measured Demand outputs. |
|  | BAResourceMeasuredDemandDeclinedHASPBidsExceptions9Flag Brt | Flag value (a binary 0-1 indicator) derived by the data mapping process from the Exceptions #9 standing data input, that indicates whether a condition from Exceptions #9 is met (true) for a given combination of Business Associate and resource ID values. When an Exceptions #9 condition is true, the BPM - CG PC Measured Demand Over Control Area shall exclude any associated metered CAISODemand, net MSS Measured Demand or export input from the calculation of the CAISOTotalHourlyMeasuredDemandMinusBalancedTOR_DeclinedHASPBidsQty mdh and BAHourlyMeasuredDemandMinusBalancedTOR_DeclinedHASPBidsQty Bmdh Measured Demand outputs. |
|  | BAMeasuredDemandDeclinedHASPBids9Flag B | Flag value (a binary 0-1 indicator) derived by the data mapping process from the Exceptions #9 standing data input, that indicates whether a condition from Exceptions #9 is met (true) for a given value for a Business Associate. When an Exceptions #9 condition is true, the BPM - CG PC Measured Demand Over Control Area shall exclude any associated metered CAISODemand, net MSS Measured Demand or export input from the calculation of the CAISOTotalHourlyMeasuredDemandMinusBalancedTOR_DeclinedHASPBidsQty mdh and BAHourlyMeasuredDemandMinusBalancedTOR_DeclinedHASPBidsQty Bmdh Measured Demand outputs. |
|  | MSSResourceInfo BrtuT’I’M’AA’VL’pmd | A flag input created by data mapping from Master File information that has a value of 1 for a MSS resource and a value of 0 for a non-MSS resource. This variable contains the information link between resource ID r and a combination of associated UDC/MSS entity u and MSS subgroup M’ values. together with values of some other UDC/MSS attributes related to u and M’. For either a UDC or MSS entity, other attributes for which values are provided include the Business Associate B, entity type T’, Aggregated Pricing Node A and Aggregated Pricing Node Type A’. For cases where T’ = ‘MSS’, u will be associated with the attributes gross/net settlement type I’, RUC Participation Flag V, and Load-Following Flag L’ that specify MSS operational or settlement selections. For a case where M’ is NULL (i.e., a MSS subgroup either does not exist for a MSS in a case where T’ = ‘MSS’ or does not apply to a UDC in a case where T’ = ‘UDC’), the other mapped attributes, if they exist, apply to u instead of u and M’. The input applies to Trading Day d of Trading Month m. |

## Inputs – Predecessor Charge Codes or Pre-calculations

Attributes that are listed for a variable name in the table below, but are not described in the associated description are incidental to and do not materially participate in the calculations of Section 3.6 herein.

| Row # | Variable Name | Predecessor Charge Code/ Pre-calc Configuration |
| --- | --- | --- |
|  | BASettlementIntervalNetMSSMeasuredDemandQuantity BuT’I’M’AA’W’VL’mdhcif | This input originates in the MSS Netting Pre-calculation. The input represents the net MSS Measured Demand Energy quantity (in MWh as a negative value) attributable to resources For the BPM - CG PC Measured Demand Over Control Area process, only input values for which T’ = ‘MSS’ and I’ = ‘NET’ are used in calculating a Measured Demand output value. |
|  | BASettlementIntervalMSSDemandQuantity_MSSNetting BuT’I’M’AA’W’VL’mdhcif | This input originates in the MSS Netting Pre-calculation. The input represents the net MSS Demand Energy quantity (in MWh as a negative value) attributable to resources for a settlement interval. In the calculations of the BPM - CG PC Measured Demand Over Control Area process, only input values for which T’ = ‘MSS’ and I’ = ‘NET’ are used. |
|  | Op_Agreement_Export_Loss_Allocation_quantity BrtEuT’I’Q’M’AA’F’R’pPW’QS’d’Nz’oVvHn’L’mdhcif | This input originates in the “Pre-Calc Allocation of Transmission Losses under Operating Agreements” pre-calc process. The input (in MWh as a negative value) represents operating agreement export losses quantity (billable) in MWh for a Settlement Interval.In the BPM - CG PC Measured Demand Over Control Area, resource type t is restricted to the value ‘ETIE’. |
|  | SettlementIntervalDeemedDeliveredInterchangeEnergyQuantity BrtEuT’I’Q’M’AA’F’R’pPW’QS’d’Nz’oVvHn’L’mdhcif | This input originates in the System Resource Deemed Delivered Energy Pre-calculation and applies to all Energy types, as denoted by the ‘E’ attribute. The derived input (in MWh as a negative value) represents Real-Time Interchange export schedule Energy as Deemed-Delivered Energy quantity in MWh for a Settlement Interval. In the BPM - CG PC Measured Demand Over Control Area output derivation, the resource type t is restricted to the value ‘ETIE’. |
|  | BAResSettlementIntervalMeteredCAISODemandQuantity BrtuT’I’Q’M’AA’R’pPW’Qd’Nz’VvHn’L’mdhcif | Received from the MSS Netting Pre-calculation, this input presents five--minute CAISO Metered Demand (in MWh as a negative value).. |
|  | BASettlementIntervalResCAISOMeteredGenerationQuantity BrtuT’I’Q’M’AA’R’pPW’Qd’Nz’VvHn’L’mdhcif | This input, an output of the MSS Netting Pre-calculation, presents five-minute metered Generation. |
|  | SettlementIntervalResourceContractMD Brtz’mdhcif | Received from the ETC/TOR/CVR Quantity Pre-calculation, the input presents the transmission right-exempted Measured Demand quantity (in MWh as a negative value) for contract type z’ for a Settlement Interval. |
|  | SettlementIntervalResourceRTContractMD Brtz’mdhcif | Received from the ETC/TORCVR Quantity Pre-calculation, this input presents the real-time transmission right-exempted Measured Demand quantity (in MWh as a negative value) for contract type z’ for a Settlement Interval. |
|  | SettlementIntervalResourceContractMDforECA BrtQ’QF’S’z’mdhcif | Received from the ETC/TOR/CVR Quantity Pre-calculation, this input presents the transmission right-exempted Measured Demand quantity (in MWh as a negative value) for contract type z’ for a Settlement Interval. |
|  | BASettlementIntervalResourceEnergyLossCreditEligibleCRNDemandQuantity Brtz’mdhcif | Received from the ETC/TOR/CVR Quantity Pre-calculation, this input presents the real-time transmission right-exempted Loss quantity (in MWh as a negative value) for contract type z’ for a Settlement Interval. . |
|  | BAHourlyResourceContractDADemandQuantity Brtz’mdh | Received from the ETC/TOR/CVR Quantity Pre-calculation, this input presents the hourly transmission right-exempted Day Ahead Demand quantity (in MW as a negative value) for contract type z’. |
|  | BAResEntitySettlementIntervalMeteredCAISODemandQuantity BrtuT’I’Q’M’AA’F’R’pPW’QS’d’Nz’VvHn’L’mdhcif | Received from the MSS Netting Pre-calculation, this input presents the five-minute CAISO Metered Demand (in MWh as a negative value) for a Business Associate and Resource.. |
|  | BAResEntitySettlementIntervalResourceFilteredCAISODemandQuantity BrtuT’I’Q’M’AA’F’R’pPW’QS’d’Nz’VvHn’L’mdhcif | Received from the MSS Netting Pre-calculation, the input represents the five-minute CAISO Metered Demand (in MWh as a negative value) for a Business Associate and Resource.. The input is filtered for Load resources where t = ‘LOAD’ or for NGR resources where ‘F’ = ‘DDR’. |

## CAISO Formula

The BPM - CG PC Measured Demand Over Control Area Pre-calculation process shall follow the requirements specified in the immediately-following subsections. Formulas presented below are intended to represent functional business requirements. Actual design implementation (for instance, representation of numbers in a computer or matrix mathematics employed) is at the discretion of designers.

    - Measured Demand Quantity over CAISO Control Area (by Trading Hour)

CAISOTotalHourlyMeasuredDemandControlAreaQty mdh = 
cf∈c CAISOTotalSettlementIntervalMeasuredDemandControlAreaQty mdhcif

    - Measured Demand Quantity over CAISO Control Area (by 10-Minute Interval)

Where

CAISOTotal10MMeasuredDemandControlAreaQty mdhi = BBA10MMeasuredDemandControlAreaQty Bmdhi

    - Measured Demand Quantity over CAISO Control Area (by Settlement Interval)

Where

CAISOTotalSettlementIntervalMeasuredDemandControlAreaQty mdhcif = BBASettlementIntervalMeasuredDemandControlAreaQty Bmdhcif

    - Measured Demand Quantity over CAISO Control Area (by BA and 10-minute interval, for Flexible Ramp Cost Allocation)

BA10MMeasuredDemandControlAreaQty Bmdhi = f∈iBASettlementIntervalMeasuredDemandControlAreaQty Bmdhcif

    - Measured Demand Quantity over CAISO Control Area (by BA and Trading Hour)

BAHourlyMeasuredDemandControlAreaQty Bmdh = cf∈cBASettlementIntervalMeasuredDemandControlAreaQty Bmdhcif

    - Measured Demand Quantity over CAISO Control Area (by BA and Settlement Interval)

Where

BASettlementIntervalMeasuredDemandControlAreaQtyBmdhcif = 
uT'I'M'W'VL'BAUDCSettlementIntervalMeasuredDemandControlAreaQty BuT’I’M’W’VL’mdhcif

    - Measured Demand Quantity over UDC Area (by Settlement Interval)

UDCTotalSettlementIntervalMeasuredDemandControlAreaQty uT’I’M’W’VL’mdhcif = BBAUDCSettlementIntervalMeasuredDemandControlAreaQty BuT’I’M’W’VL’mdhcif

    - Measured Demand Quantity over UDC Area (by BA and Trading Hour)

Where

BAUDCHourlyMeasuredDemandControlAreaQty BuT’I’M’W’VL’mdh = cf∈cBAUDCSettlementIntervalMeasuredDemandControlAreaQty BuT’I’M’W’VL’mdhcif

    - Measured Demand Quantity over UDC Area  (by BA and Settlement Interval)

Where

BAUDCSettlementIntervalMeasuredDemandControlAreaQty BuT’I’M’W’VL’mdhcif = BASettlementIntervalUDCTotalMeteredCAISODemandQuantity_MDOverCA BuT’I’M’W’VL’mdhcif 
+ BASettlementIntervalUDCTotalNetMSSMeasuredDemandQty_MDOverCA BuT’I’M’W’VL’mdhcif
+ BASettlementIntervalUDCExportQuantity_MDOverCA BuT’I’M’W’VL’mdhcif

      - Where

BASettlementIntervalUDCTotalMeteredCAISODemandQuantity_MDOverCA BuT’I’M’W’VL’mdhcif =

AA'BASettlementIntervalUDCNodalMeteredCAISODemandQuantity BuT’I’M’AA’W’VL’mdhcif

        - Where

BASettlementIntervalUDCNodalMeteredCAISODemandQuantity BuT’I’M’AA’W’VL’mdhcif

= rtQ'R'pPQd'Nz'vHn'BAResSettlementIntervalMeteredCAISODemandQuantity BrtuT’I’Q’M’AA’R’pPW’Qd’Nz’VvHn’L’mdhcif

        - And Where (expressed explicitly for formula 3.6.9.1, but not formula 3.6.9.1.1)

(

T’ = ‘UDC’

        - Or Where

T’ = ‘MSS’ And I’ = ’GROSS’

)

      - And Where

BASettlementIntervalUDCTotalNetMSSMeasuredDemandQty_MDOverCA BuT’I’M’W’VL’mdhcif = 
AA'BASettlementIntervalNetMSSMeasuredDemandQuantity BuT’I’M’AA’W’VL’mdhcif

        - Where

T’ = ‘MSS’

        - And Where

I’ = ‘NET’

      - And Where

BASettlementIntervalUDCExportQuantity_MDOverCA BuT’I’M’W’VL’mdhcif = 
rtE Q'AA'F'R'pPQS'd'Nz'OvHn' (SettlementIntervalDeemedDeliveredInterchangeEnergyQuantity BrtEuT’I’Q’M’AA’F’R’pPW’QS’d’Nz’oVvHn’L’mdhcif + Op_Agreement_Export_Loss_Allocation_Quantity BrtEuT’I’Q’M’AA’F’R’pPW’QS’d’Nz’oVvHn’L’mdhcif)

        - Where

t = ‘ETIE’ and Q’ = ‘CISO’

        - And Where

E In ( ‘FIRM’, ‘NFRM’, ‘WHEEL’, ‘DYN’, ‘UCTG’)

        - And Where

(

T’ = ‘UDC’

        - Or Where

T’ = ‘MSS’ And I’ = ’GROSS’

)

    - Metered CAISO Demand Quantity (by Aggregated Price Node and Trading Hour)

HourlyNodalMeteredCAISODemandQuantity_MDOverCA AA’mdh = 
cf∈cSum over (u,M’,c,i,f) {SettlementIntervalNodalMeteredCAISODemandQuantity_MDOverCA uM’AA’mdhcif }

    - Metered CAISO Demand Quantity (by Aggregated Price Node and Settlement Interval)

Where

    - SettlementIntervalNodalMeteredCAISODemandQuantity_MDOverCA uM’AA’mdhcif = 
B BAEntitySettlementIntervalNodalMeteredCAISODemandQuantity BuM’AA’mdhcif

      - Where

- BAEntitySettlementIntervalNodalMeteredCAISODemandQuantity BuM’AA’mdhcif =

Sum over (r,t,T’,I’,Q’, F’,R’,p,P,W’,Q,S’,d’,N,z’,V,v,H,n’,L’) {BAResEntitySettlementIntervalMeteredCAISODemandQuantity BrtuT’I’Q’M’AA’F’R’pPW’QS’d’Nz’VvHn’L’mdhcif }

Where Resource type (t) = ‘LOAD’ and Entity Component Subtype (S’) = ‘NPL’ OR ’GL’

      - Where

- Metered CAISO Demand Quantity (by BA, Aggregated Price Node and Trading Hour)

BAHourlyNodalMeteredCAISODemandQuantity_MDOverCA BAA’mdh = 
cf∈c BASettlementIntervalNodalMeteredCAISODemandQuantity_MDOverCA BAA’mdhcif

    - Metered CAISO Demand Quantity (by BA, Aggregated Price Node and Settlement Interval)

Where

BASettlementIntervalNodalMeteredCAISODemandQuantity_MDOverCA BAA’mdhcif =

BASettlementIntervalNodalNetMSSDemandQuantity BAA’mdhcif +  BASettlementInterval_NodalMeteredCAISODemandQuantity BAA’mdhcif

      - Where

BASettlementIntervalNodalNetMSSDemandQuantity BAA’mdhcif =

uT'I'M'W'VL' BASettlementIntervalMSSDemandQuantity_MSSNetting BuT’I’M’AA’W’VL’mdhcif

        - Where

T’ = ‘MSS’

        - And Where

I’ = ‘NET’

      - And Where

BASettlementInterval_NodalMeteredCAISODemandQuantity BAA’mdhcif =

uT'I'M'W'VL' BASettlementIntervalUDCNodalMeteredCAISODemandQuantity BuT’I’M’AA’W’VL’mdhcif

        - Where

(

T’ = ‘UDC’

        - Or Where

T’ = ‘MSS’ And I’ = ’GROSS’

)

    - Measured Demand Quantity Excluding All Transmission Rights over CAISO Control Area (by Trading Hour)

CAISOTotalHourlyMeasuredDemandMinusRightsControlAreaQty mdh = 
cf∈c CAISOTotalSettlementIntervalMeasuredDemandMinusRightsControlAreaQty mdhcif

    - Measured Demand Quantity Excluding Transmission Rights over CAISO Control Area (by Settlement Interval)

Where

CAISOTotalSettlementIntervalMeasuredDemandMinusRightsControlAreaQty mdhcif = BBASettlementIntervalMeasuredDemandMinusRightsControlAreaQty Bmdhcif

    - Measured Demand Quantity Excluding Transmission Rights over CAISO Control Area (by BA and Trading Hour)

BAHourlyMeasuredDemandMinusRightsControlAreaQty Bmdh = cf∈cBASettlementIntervalMeasuredDemandMinusRightsControlAreaQty Bmdhcif

      - Where

BASettlementIntervalMeasuredDemandMinusRightsControlAreaQty Bmdhcif = 
Min(0, BASettlementIntervalMeasuredDemandControlAreaQty Bmdhcif–- BASettlementIntervalRightsMDControlAreaQty Bmdhcif)

        - Where

BASettlementIntervalRightsMDControlAreaQty Bmdhcif =  rtz'SettlementIntervalResourceContractMD Brtz’mdhcif

    - Measured Demand Quantity Excluding Real-Time Transmission Rights over CAISO Control Area (by Trading Hour)

CAISOTotalHourlyMeasuredDemandMinusRTRightsControlAreaQty mdh = 
cf∈c CAISOTotalSettlementIntervalMeasuredDemandMinusRTRightsControlAreaQty mdhcif

    - Measured Demand Quantity Excluding Real-Time Transmission Rights over CAISO Control Area (by Settlement Interval)

Where

CAISOTotalSettlementIntervalMeasuredDemandMinusRTRightsControlAreaQty mdhcif = BBASettlementIntervalMeasuredDemandMinusRTRightsControlAreaQty Bmdhcif

    - Measured Demand Quantity Excluding Real-Time Transmission Rights over CAISO Control Area (by BA and Trading Hour)

BAHourlyMeasuredDemandMinusRTRightsControlAreaQty Bmdh = cf∈cBASettlementIntervalMeasuredDemandMinusRTRightsControlAreaQty Bmdhcif

      - Where

BASettlementIntervalMeasuredDemandMinusRTRightsControlAreaQty Bmdhcif = BASettlementIntervalMeasuredDemandControlAreaQty Bmdhcif – BASettlementIntervalRTRights_MDControlAreaQty Bmdhcif

        - Where

BASettlementIntervalRTRights_MDControlAreaQty Bmdhcif =  rtz'SettlementIntervalResourceRTContractMD Brtz’mdhcif

          - Where

z’ = ‘ETC’ Or z’ = ‘TOR’

    - Measured Demand Quantity Excluding Real-Time TOR Transmission Rights over CAISO Control Area (by Trading Hour)

CAISOTotalHourlyMeasuredDemandMinusRT_TORControlAreaQty mdh = 
cf∈c CAISOTotalSettlementIntervalMeasuredDemandMinusRT_TORControlAreaQty mdhcif

    - Measured Demand Quantity Excluding Real-Time TOR Transmission Rights over CAISO Control Area (by Settlement Interval)

Where

CAISOTotalSettlementIntervalMeasuredDemandMinusRT_TORControlAreaQty mdhcif = BBASettlementIntervalMeasuredDemandMinusRT_TORControlAreaQty Bmdhcif

    - Measured Demand Quantity Excluding Real-Time TOR Transmission Rights over CAISO Control Area (by BA and Trading Hour)

BAHourlyMeasuredDemandMinusRT_TORControlAreaQty Bmdh = cf∈cBASettlementIntervalMeasuredDemandMinusRT_TORControlAreaQty Bmdhcif

      - Where

BASettlementIntervalMeasuredDemandMinusRT_TORControlAreaQty Bmdhcif = BASettlementIntervalMeasuredDemandControlAreaQty Bmdhcif – BASettlementIntervalRT_TOR_MDControlAreaQty Bmdhcif

        - Where

BASettlementIntervalRT_TOR_MDControlAreaQty Bmdhcif =  rtz'SettlementIntervalResourceRTContractMD Brtz’mdhcif

          - Where

z’ = ‘TOR’

    - Measured Demand Quantity Excluding Transmission Rights over CAISO Control Area – Exceptions #1 (by Trading Hour)

CAISOTotalHourlyMeasuredDemandMinusRightsControlAreaQty_Ex1 mdh = 
i CAISOTotal10MMeasuredDemandMinusRightsControlAreaQty_Ex1 mdhi

    - Measured Demand Quantity Excluding Transmission Rights over CAISO Control Area – Exceptions #1 (by Settlement Interval)

Where

CAISOTotal10MMeasuredDemandMinusRightsControlAreaQty_Ex1 mdhi = BBA10MMeasuredDemandMinusRightsControlAreaQtyEx1 Bmdhi

    - Measured Demand Quantity Excluding Transmission Rights over CAISO Control Area – Exceptions #1 (by BA and Trading Hour)

BAHourlyMeasuredDemandMinusRightsControlAreaQty_Ex1 Bmdh = iBA10MMeasuredDemandMinusRightsControlAreaQty_Ex1 Bmdhi

    - Measured Demand Quantity Excluding Transmission Rights over CAISO Control Area – Exceptions #1 (by BA and Settlement Interval)

Where

BA10MMeasuredDemandMinusRightsControlAreaQty_Ex1 Bmdhi = 
f∈iMin(0, BASettlementIntervalMeasuredDemandControlAreaQty_Ex1 Bmdhcif – BASettlementIntervalRightsMDControlAreaQty_Ex5 Bmdhcif)

      - Where

If MeasuredDemandControlAreaExceptions5Flag B =  ‘True’

Then
(

        - BASettlementIntervalRightsMDControlAreaQty_Ex5 Bmdhcif =  rtz'SettlementIntervalResourceContractMD Brtz’mdhcif

)

Else

(

        - BASettlementIntervalRightsMDControlAreaQty_Ex5 Bmdhcif =  0

)

    - Measured Demand Quantity over CAISO Control Area – Exceptions #1 (by Trading Hour)

CAISOTotalHourlyMeasuredDemandControlAreaQty_Ex1 mdh = 
cf∈c CAISOTotalSettlementIntervalMeasuredDemandControlAreaQty_Ex1 mdhcif

    - Measured Demand Quantity over CAISO Control Area – Exceptions #1 (by Settlement Interval)

Where

CAISOTotalSettlementIntervalMeasuredDemandControlAreaQty_Ex1 mdhcif = BBASettlementIntervalMeasuredDemandControlAreaQty_Ex1 Bmdhcif

    - Measured Demand Quantity over CAISO Control Area – Exceptions #1 (by BA and Trading Hour)

BAHourlyMeasuredDemandControlAreaQty_Ex1 Bmdh = cf∈cBASettlementIntervalMeasuredDemandControlAreaQty_Ex1 Bmdhcif

    - Measured Demand Quantity over CAISO Control Area – Exceptions #1 (by BA and Settlement Interval)

Where

BASettlementIntervalMeasuredDemandControlAreaQty_Ex1 Bmdhcif = 
uT'I'M'W'VL'BAUDCSettlementIntervalMeasuredDemandControlAreaQty_Ex1 BuT’I’M’W’VL’mdhcif

    - Measured Demand Quantity over UDC Area – Exceptions #1 (by Settlement Interval)

UDCTotalSettlementIntervalMeasuredDemandControlAreaQty_Ex1 uT’I’M’W’VL’mdhcif = BBAUDCSettlementIntervalMeasuredDemandControlAreaQty_Ex1 BuT’I’M’W’VL’mdhcif

    - Measured Demand Quantity over UDC Area – Exceptions #1 (by BA and Trading Hour)

Where

BAUDCHourlyMeasuredDemandControlAreaQty_Ex1 BuT’I’M’W’VL’mdh = cf∈cBAUDCSettlementIntervalMeasuredDemandControlAreaQty_Ex1 BuT’I’M’W’VL’mdhcif

    - Measured Demand Quantity over UDC Area – Exceptions #1 (by BA and Settlement Interval)

Where

BAUDCSettlementIntervalMeasuredDemandControlAreaQty_Ex1 BuT’I’M’W’VL’mdhcif = BASettlementIntervalUDCTotalMeteredCAISODemandQuantity_Ex1_MDOverCA BuT’I’M’W’VL’mdhcif 
+ BASettlementIntervalUDCTotalNetMSSMeasuredDemandQty_Ex1_MDOverCA BuT’I’M’W’VL’mdhcif 
+ BASettlementIntervalUDCExportQuantity_Ex1_MDOverCA BuT’I’M’W’VL’mdhcif

      - Where

BASettlementIntervalUDCTotalMeteredCAISODemandQuantity_Ex1_MDOverCA BuT’I’M’W’VL’mdhcif = rtQ'AA'R'pPQd'Nz'vHn'BAResSettlementIntervalMeteredCAISODemandQuantity BrtuT’I’Q’M’AA’R’pPW’Qd’Nz’VvHn’L’mdhcif

        - Where

(

T’ = ‘UDC’

        - Or Where

T’ = ‘MSS’ And I’ = ’GROSS’

)

        - And Where

MeasuredDemandControlAreaExceptions1Flag Brt <> True

        - And Where

BAMeasuredDemandCAMDExceptionsFlag B<> True

      - And Where

BASettlementIntervalUDCTotalNetMSSMeasuredDemandQty_Ex1_MDOverCA BuT’I’M’W’VL’mdhcif = 
AA'BASettlementIntervalNetMSSMeasuredDemandQuantity BuT’I’M’AA’W’VL’mdhcif

        - Where

T’ = ‘MSS’

        - And Where

I’ = ‘NET’

        - And Where

BAMeasuredDemandCAMDExceptionsFlag B <> True

      - And Where

BASettlementIntervalUDCExportQuantity_Ex1_MDOverCA BuT’I’M’W’VL’mdhcif = 
rtE Q'AA'F'R'pPQS'd'Nz'OvHn' (SettlementIntervalDeemedDeliveredInterchangeEnergyQuantity BrtEuT’I’Q’M’AA’F’R’pPW’QS’d’Nz’oVvHn’L’mdhcif + Op_Agreement_Export_Loss_Allocation_Quantity BrtEuT’I’Q’M’AA’F’R’pPW’QS’d’Nz’oVvHn’L’mdhcif)

        - Where

t = ‘ETIE’ and Q’ = ‘CISO’

        - And Where

E In ( ‘FIRM’, ‘NFRM’, ‘WHEEL’, ‘DYN’, ‘UCTG’)

        - And Where

(

T’ = ‘UDC’

        - Or Where

T’ = ‘MSS’ And I’ = ’GROSS’

)

        - And Where

MeasuredDemandControlAreaExceptions1Flag Brt <> True

        - And Where

BAMeasuredDemandCAMDExceptionsFlag B<> True

    - Gross Metered Demand Quantity over UDC Area, Excluding Transmission Rights – Exceptions #1, UFE (by Settlement Interval)

UDCTotalSettlementIntervalGrossMeteredDemandControlAreaQty_Ex1 uT’I’M’W’VL’mdhcif = BBAUDCSettlementIntervalGrossMeteredDemandControlAreaQty_Ex1 BuT’I’M’W’VL’mdhcif

    - Gross Metered Demand Quantity over UDC Area, Excluding Transmission Rights – Exceptions #1, UFE (by BA and Trading Hour)

Where

BAUDCHourlyGrossMeteredDemandControlAreaQty_Ex1 BuT’I’M’W’VL’mdh = cf∈cBAUDCSettlementIntervalGrossMeteredDemandControlAreaQty_Ex1 BuT’I’M’W’VL’mdhcif

    - Gross Metered Demand Quantity over UDC Area, Excluding Transmission Rights – Exceptions #1, UFE (by BA and Settlement Interval)

Where

BAUDCSettlementIntervalGrossMeteredDemandControlAreaQty_Ex1 BuT’I’M’W’VL’mdhcif = BASettlementIntervalUDCTotalMeteredCAISODemandQuantity_Ex1_MDOverCA BuT’I’M’W’VL’mdhcif 
+ BASettlementIntervalUDCTotalNetMSSGrossLoadQty_Ex1_MDOverCA BuT’I’M’W’VL’mdhcif – BASettlementIntervalMeteredDemandRightsMDControlAreaQty_UFE BuT’I’M’Vmdhcif

      - Where

BASettlementIntervalUDCTotalNetMSSGrossLoadQty_Ex1_MDOverCA BuT’I’M’W’VL’mdhcif = 
rtQ'AA'R'pPQd'Nz'vHn' BAResSettlementIntervalMeteredCAISODemandQuantity BrtuT’I’Q’M’AA’R’pPW’Qd’Nz’VvHn’L’mdhcif

        - Where

T’ = ‘MSS’

        - And Where

I’ = ‘NET’

        - And Where

MeasuredDemandControlAreaExceptions1Flag Brt <> True

        - And Where

BAMeasuredDemandCAMDExceptionsFlag B<> True

      - And Where

BASettlementIntervalMeteredDemandRightsMDControlAreaQty_UFE BuT’I’M’Vmdhcif =  BASettlementIntervalTORMeteredDemand Bmdhcif + EntityBasicInfo BuT’I’M’Vmd

        - Where

BAMeasuredDemandCAMDExceptionsFlag B<> True

        - And Where

EntityBasicInfo BuT’I’M’Vmd = rtAA'pL'0 * MSSResourceInfo BrtuT’I’M’AA’VL’pmd

          - Where

t = ‘LOAD’

          - And Where

MeasuredDemandControlAreaExceptions1Flag Brt <> True

          - And Where

BAMeasuredDemandCAMDExceptionsFlag B<> True

        - And Where

BASettlementIntervalTORMeteredDemand Bmdhcif = rtz'SettlementIntervalResourceContractMD Brtz’mdhcif

          - Where

t <> ‘ETIE’

          - And Where

z’ = ‘TOR’

          - And Where

MeasuredDemandControlAreaExceptions1Flag Brt <> True

          - And Where

BAMeasuredDemandCAMDExceptionsFlag B <> True

    - Gross Measured Demand Quantity over UDC Area – Exceptions #1 (by Settlement Interval)

UDCTotalSettlementIntervalGrossMeasuredDemandControlAreaQty_Ex1 uT’I’M’W’VL’mdhcif = BBAUDCTotalSettlementIntervalGrossMeasuredDemandControlAreaQty_Ex1 BuT’I’M’W’VL’mdhcif

    - Gross Measured Demand Quantity over UDC Area – Exceptions #1 (by BA and Settlement Interval)

Where

BAUDCTotalSettlementIntervalGrossMeasuredDemandControlAreaQty_Ex1 BuT’I’M’W’VL’mdhcif = BASettlementIntervalUDCTotalMeteredCAISODemandQuantity_Ex1_MDOverCA BuT’I’M’W’VL’mdhcif 
+ BASettlementIntervalNetSettledMSSTotalGrossMeasuredDemandQty_Ex1_MDOverCA BuT’I’M’W’VL’mdhcif 
+ BASettlementIntervalUDCExportQuantity_Ex1_MDOverCA BuT’I’M’W’VL’mdhcif

    - Total Gross Measured Demand Quantity over Net-Settled MSS Area – Exceptions #1 (by BA and Settlement Interval)

Where

BASettlementIntervalNetSettledMSSTotalGrossMeasuredDemandQty_Ex1_MDOverCA BuT’I’M’W’VL’mdhcif = 
BASettlementIntervalNetSettledMSSTotalMeteredCAISODemandQuantity_Ex1_MDOverCA BuT’I’M’W’VL’mdhcif +  BASettlementIntervalNetSettledMSSExportQuantity_Ex1_MDOverCA BuT’I’M’W’VL’mdhcif

      - Where

BASettlementIntervalNetSettledMSSTotalMeteredCAISODemandQuantity_Ex1_MDOverCA BuT’I’M’W’VL’mdhcif = rtQ'AA'R'pPQd'Nz'vHn'BAResSettlementIntervalMeteredCAISODemandQuantity BrtuT’I’Q’M’AA’R’pPW’Qd’Nz’VvHn’L’mdhcif

        - Where

T’ = ‘MSS’ And I’ = ‘NET’

        - And Where

MeasuredDemandControlAreaExceptions1Flag Brt <> True

        - And Where

BAMeasuredDemandCAMDExceptionsFlag B<> True

      - And Where

BASettlementIntervalNetSettledMSSExportQuantity_Ex1_MDOverCA BuT’I’M’W’VL’mdhcif = 
rtE Q'AA'F'R'pPQS'd'Nz'OvHn' (SettlementIntervalDeemedDeliveredInterchangeEnergyQuantity BrtEuT’I’Q’M’AA’F’R’pPW’QS’d’Nz’oVvHn’L’mdhcif + Op_Agreement_Export_Loss_Allocation_Quantity BrtEuT’I’Q’M’AA’F’R’pPW’QS’d’Nz’oVvHn’L’mdhcif)

        - Where

T’ = ‘MSS’ And I’ = ‘NET’

        - And Where

t = ‘ETIE’

        - And Where

E In ( ‘FIRM’, ‘NFRM’, ‘WHEEL’, ‘DYN’, ‘UCTG’)

        - And Where

MeasuredDemandControlAreaExceptions1Flag Brt <> True

        - And Where

BAMeasuredDemandCAMDExceptionsFlag B <> True

-- Metered and Measured Demand Charge Types for FERC Fees–-

    - Measured Demand Quantity over CAISO Control Area for FERC Exceptions (by Settlement Trading Day)

CAISOTotalDailyMeasuredDemandControlAreaQty_FERC md = 
hcif CAISOTotalSettlementIntervalMeasuredDemandControlAreaQty_FERC mdhcif

    - Measured Demand Quantity over CAISO Control Area for FERC Exceptions (by Settlement Interval)

Where

CAISOTotalSettlementIntervalMeasuredDemandControlAreaQty_FERC mdhcif = BBASettlementIntervalMeasuredDemandControlAreaQty_FERC Bmdhcif

    - Measured Demand Quantity over CAISO Control Area for FERC Exceptions (by BA and Settlement Trading Day)

BADailyMeasuredDemandControlAreaQty_FERC Bmd = hcifBASettlementIntervalMeasuredDemandControlAreaQty_FERC Bmdhcif

    - Measured Demand Quantity over CAISO Control Area for FERC Exceptions (by BA and Settlement Interval)

Where

BASettlementIntervalMeasuredDemandControlAreaQty_FERC Bmdhcif= Min (0, BASettlementIntervalTotalMeteredCAISODemandQuantity_FERC Bmdhcif + BASettlementIntervalTotalNetMSSMeasuredDemandQty_FERC Bmdhcif+ BASettlementIntervalExportQuantity_FERCBmdhcif–- BASettlementIntervalRightsMDControlAreaQty_Ex5 Bmdhcif + BASettlementIntervalGenerationQuantity_FERC Bmdhcif)

      - Where

BASettlementIntervalTotalMeteredCAISODemandQuantity_FERC Bmdhcif = rtuT'I'Q'M'AA'R'pPW'Qd'Nz'VvHn'L'BAResSettlementIntervalMeteredCAISODemandQuantity BrtuT’I’Q’M’AA’R’pPW’Qd’Nz’VvHn’L’mdhcif

        - Where

(

T’ = ‘UDC’

        - Or Where

T’ = ‘MSS’ And I’ = ’GROSS’

)

        - And Where

MeasuredDemandControlAreaExceptions2Flag Brt <> True

        - And Where

BAMeasuredDemandFERCExceptionsFlag B <> True

      - And Where

BASettlementIntervalTotalNetMSSMeasuredDemandQty_FERC Bmdhcif = 
uM'AA'T'I'W'VL'BASettlementIntervalNetMSSMeasuredDemandQuantity BuT’I’M’AA’W’VL’mdhcif

        - Where

T’ = ‘MSS’

        - And Where

I’ = ‘NET’

        - And Where

BAMeasuredDemandFERCExceptionsFlag B <> True

      - And Where

BASettlementIntervalExportQuantity_FERC Bmdhcif = 
rtEuT'I' Q'M'AA'F'R'pPW'QS'd'Nz'OVvHn'L' (SettlementIntervalDeemedDeliveredInterchangeEnergyQuantity BrtEuT’I’Q’M’AA’F’R’pPW’QS’d’Nz’oVvHn’L’mdhcif + Op_Agreement_Export_Loss_Allocation_Quantity BrtEuT’I’Q’M’AA’F’R’pPW’QS’d’Nz’oVvHn’L’mdhcif)

        - Where

t = ‘ETIE’ and Q’ = ‘CISO’

        - And Where

E In (‘FIRM’, ‘NFRM’, ‘WHEEL’, ‘DYN’, ‘UCTG’)

        - And Where

(

T’ = ‘UDC’

        - Or Where

T’ = ‘MSS’ And I’ = ’GROSS’

)

        - And Where

MeasuredDemandControlAreaExceptions2Flag Brt <> True

        - And Where

BAMeasuredDemandFERCExceptionsFlag B <> True

      - And Where

BASettlementIntervalGenerationQuantity_FERC Bmdhcif = rtuT'I'Q'M'AA'R'pPW'Qd'VNz'vHn'L'BASettlementIntervalResCAISOMeteredGenerationQuantity BrtuT’I’Q’M’AA’R’pPW’Qd’Nz’VvHn’L’mdhcif

        - Where

(

T’ = ‘UDC’

        - Or Where

T’ = ‘MSS’ And I’ = ’GROSS’

)

        - And Where

MeasuredDemandControlAreaExceptions3Flag Brt = True

        - And Where

BAMeasuredDemandGenFERCExceptionsFlag B  = True

-- Gross Metered and Gross Measured Demand Charge Types  --

    - Gross Measured Quantity over Control Area – BCR (by Trading Hour)

CAISOHourlyDAGrossMeasuredDemand mdh = 
BrtuT'I'Q'M'AA'R'pPW'Qd'Nz' VvHn'L'cf∈c(BAResIntervalUDCMeteredCAISODemandQuantity BrtuT’I’Q’M’AA’R’pPW’Qd’Nz’VvHn’L’mdhcif + BAResIntervalGrossMSSMeteredCAISODemandQuantity BrtuT’I’Q’M’AA’R’pPW’Qd’Nz’VvHn’L’mdhcif + BAResIntervalNetMSSMeteredCAISODemandQuantity BrtuT’I’Q’M’AA’R’pPW’Qd’Nz’VvHn’L’mdhcif + SettlementIntervalDeemedDeliveredInterchangeEnergyExportTotalQuantity BrtuT’I’Q’M’AA’R’pPW’Qd’Nz’VvHn’L’mdhcif)

    - Metered Demand Quantity for UDC Entities – BCR (by BA, Resource and Settlement Interval)

Where

BAResIntervalUDCMeteredCAISODemandQuantity BrtuT’I’Q’M’AA’R’pPW’Qd’Nz’VvHn’L’mdhcif 
= F'S'BAResEntitySettlementIntervalMeteredCAISODemandQuantity BrtuT’I’Q’M’AA’F’R’pPW’QS’d’Nz’VvHn’L’mdhcif

      - Where

T’ = ‘UDC’

    - Metered Demand Quantity for MSS Gross-Settled Entities – BCR (by BA, Resource and Settlement Interval)

Where

BAResIntervalGrossMSSMeteredCAISODemandQuantity BrtuT’I’Q’M’AA’R’pPW’Qd’Nz’VvHn’L’mdhcif 
=

BAResSettlementIntervalMeteredCAISODemandQuantity BrtuT’I’Q’M’AA’R’pPW’Qd’Nz’VvHn’L’mdhcif

      - Where

T’ = ‘MSS’ And I’ = ’GROSS’

    - Metered Demand Quantity for MSS Net-Settled Entities – BCR (by BA, Resource and Settlement Interval)

Where

BAResIntervalNetMSSMeteredCAISODemandQuantity BrtuT’I’Q’M’AA’R’pPW’Qd’Nz’VvHn’L’mdhcif = F'S'BAResEntitySettlementIntervalMeteredCAISODemandQuantity BrtuT’I’Q’M’AA’F’R’pPW’QS’d’Nz’VvHn’L’mdhcif

      - Where

S’ = ‘GL’

      - And Where

T’ = ‘MSS’ And I’ = ’NET’

-- Metered and Measured Demand Charge Types for BCR–-

    - Gross Metered Demand Quantity for Resource – BCR (by BA and Trading Hour)

BAHourlyResMeteredDemandControlAreaQty_BCR BrtuT’I’M’F’W’S’VL’mdh = cf∈cBASettlementIntervalResMeteredDemandControlAreaQty_BCR BrtuT’I’M’F’W’S’VL’mdhcif

      - Where

MeasuredDemandControlAreaExceptions6Flag Brt <> True

      - And Where

BAMeasuredDemandBCRExceptionsFlag B <> True

    - Gross Metered Demand Quantity for Resource, Excluding TOR Transmission Rights for UDC and MSS Entites – BCR (by BA and Trading Hour)

BAHourlyResMeteredDemandMinusTORControlAreaQty_BCR BrtuT’I’M’F’W’S’VL’mdh = cf∈c BASettlementIntervalResMeteredDemandMinusTORControlAreaQty_BCR BrtuT’I’M’F’W’S’VL’mdhcif

    - Gross Metered Demand Quantity for Load Resource, Excluding TOR Transmission Rights for UDC and MSS Entites – BCR (by BA and Settlement Interval)

Where

BASettlementIntervalResMeteredDemandMinusTORControlAreaQty_BCR BrtuT’I’M’F’W’S’VL’mdhcif = Min (0, BASettlementIntervalResMeteredDemandControlAreaQty_BCR BrtuT’I’M’F’W’S’VL’mdhcif–- BASettlementIntervalResTOR_MDControlAreaQty_BCR BrtF’S’mdhcif)

      - Where

t <> ‘ETIE’

      - And Where

MeasuredDemandControlAreaExceptions6Flag Brt <> True

      - And Where

BAMeasuredDemandBCRExceptionsFlag B <> True

      - And Where

If z’ = ‘TOR’

        - Then

BASettlementIntervalResTOR_MDControlAreaQty_BCR BrtF’S’mdhcif = Q'Qz'SettlementIntervalResourceContractMDforECA BrtQ’QF’S’z’mdhcif

        - Else

BASettlementIntervalResTOR_MDControlAreaQty_BCR BrtF’S’mdhcif = 0

End If

      - Gross Metered Demand Quantity for Resource – BCR (by BA and Settlement Interval)

And Where

BASettlementIntervalResMeteredDemandControlAreaQty_BCR BrtuT’I’M’F’W’S’VL’mdhcif = Q'AA'R'pPQd'Nz'vHn' BAResSettlementIntervalUDCMeteredCAISODemandQuantity BrtuT’I’Q’M’AA’F’R’pPW’QS’d’Nz’VvHn’L’mdhcif + BAResSettlementIntervalGrossMSSMeteredCAISODemandQuantity BrtuT’I’Q’M’AA’F’R’pPW’QS’d’Nz’VvHn’L’mdhcif + BAResSettlementIntervalNetMSSMeteredCAISODemandQuantity BrtuT’I’Q’M’AA’F’R’pPW’QS’d’Nz’VvHn’L’mdhcif

      - Where

BAResSettlementIntervalUDCMeteredCAISODemandQuantity BrtuT’I’Q’M’AA’F’R’pPW’QS’d’Nz’VvHn’L’mdhcif = BAResEntitySettlementIntervalMeteredCAISODemandQuantity BrtuT’I’Q’M’AA’F’R’pPW’QS’d’Nz’VvHn’L’mdhcif

        - Where

T’ = ‘UDC’

      - And Where

BAResSettlementIntervalGrossMSSMeteredCAISODemandQuantity BrtuT’I’Q’M’AA’F’R’pPW’QS’d’Nz’VvHn’L’mdhcif = BAResEntitySettlementIntervalResourceFilteredCAISODemandQuantity BrtuT’I’Q’M’AA’F’R’pPW’QS’d’Nz’VvHn’L’mdhcif

        - Where

T’ = ‘MSS’ And I’ = ‘GROSS’

      - And Where

BAResSettlementIntervalNetMSSMeteredCAISODemandQuantity BrtuT’I’Q’M’AA’F’R’pPW’QS’d’Nz’VvHn’L’mdhcif = BAResEntitySettlementIntervalMeteredCAISODemandQuantity BrtuT’I’Q’M’AA’F’R’pPW’QS’d’Nz’VvHn’L’mdhcif

        - Where

S’ = ‘GL’

        - And Where

T’ = ‘MSS’ And I’ = ‘NET’

    - Real Time TOR Transmission Rights for Loads of UDC and MSS Entites with Exemptions #6 – BCR (by BA and Trading Hour)

BAHourlyResRealTimeTORLoadQty_Ex6_BCR Brmdh = tQ'QF'S'z'cf∈cSettlementIntervalResourceContractMDforECA BrtQ’QF’S’z’mdhcif

      - Where

t <> ‘ETIE’

      - And Where

z’ = ‘TOR’

      - And Where

MeasuredDemandControlAreaExceptions6Flag Brt <> True

      - And Where

BAMeasuredDemandBCRExceptionsFlag B <> True

    - Day Ahead TOR Transmission Rights for Loads of UDC and MSS Entites with Exemptions #6 – BCR (by BA and Trading Hour)

BAHourlyResDayAheadTORLoadQty_Ex6_BCR Brmdh = tz'BAHourlyResourceContractDADemandQuantity Brtz’mdh

      - Where

t <> ‘ETIE’

      - And Where

z’ = ‘TOR’

      - And Where

MeasuredDemandControlAreaExceptions6Flag Brt <> True

      - And Where

BAMeasuredDemandBCRExceptionsFlag B <> True

-- BCR-Related Measured Demand for Load-Following MSS Entities–-

    - Measured Demand Quantity Excluding Transmission Rights over CAISO Control Area – Load-Following Net Settlement and Exceptions #6 (by Trading Hour)

CAISOTotalHourlyMeasuredDemandMinusRightsControlAreaQty_LFEx6 mdh = 
cf∈cCAISOTotalSettlementIntervalMeasuredDemandMinusRightsControlAreaQty_LFEx6 mdhcif

    - Measured Demand Quantity Excluding Transmission Rights over CAISO Control Area – Load-Following Net Settlement and Exceptions #6 (by Settlement Interval)

Where

CAISOTotalSettlementIntervalMeasuredDemandMinusRightsControlAreaQty_LFEx6 mdhcif = BBASettlementIntervalMeasuredDemandMinusRightsControlAreaQty_LFEx6 Bmdhcif

    - Measured Demand Quantity Excluding Transmission Rights over CAISO Control Area – Load-Following Net Settlement and Exceptions #6 (by BA and Trading Hour)

BAHourlyMeasuredDemandMinusRightsControlAreaQty_LFEx6 Bmdh = cf∈cBASettlementIntervalMeasuredDemandMinusRightsControlAreaQty_LFEx6 Bmdhcif

    - Measured Demand Quantity Excluding Transmission Rights over CAISO Control Area – Load-Following Net Settlement and Exceptions #6 (by BA and Settlement Interval)

Where

BASettlementIntervalMeasuredDemandMinusRightsControlAreaQty_LFEx6 Bmdhcif = 
MIN(0, BASettlementIntervalMeasuredDemandControlAreaQty_LFEx6 Bmdhcif – BASettlementIntervalTOR_MDControlAreaQty_BCR Bmdhcif)

      - Where

BAMeasuredDemandBCRExceptionsFlag B  <> True

      - And Where

BASettlementIntervalTOR_MDControlAreaQty_BCR Bmdhcif = rtF'S'BASettlementIntervalResTOR_MDControlAreaQty_BCR BrtF’S’mdhcif

        - Where

MeasuredDemandControlAreaExceptions6Flag Brt <> True

    - Measured Demand Quantity over CAISO Control Area – Load-Following Net Settlement and Exceptions #6 (by Trading Hour)

CAISOTotalHourlyMeasuredDemandControlAreaQty_LFEx6 mdh = 
cf∈c CAISOTotalSettlementIntervalMeasuredDemandControlAreaQty_LFEx6 mdhcif

    - Measured Demand Quantity over CAISO Control Area – Load-Following Net Settlement and Exceptions #6 (by Settlement Interval)

Where

CAISOTotalSettlementIntervalMeasuredDemandControlAreaQty_LFEx6 mdhcif = BBASettlementIntervalMeasuredDemandControlAreaQty_LFEx6 Bmdhcif

    - Measured Demand Quantity over CAISO Control Area – Load-Following Net Settlement and Exceptions #6 (by BA and Trading Hour)

BAHourlyMeasuredDemandControlAreaQty_LFEx6 Bmdh = cf∈cBASettlementIntervalMeasuredDemandControlAreaQty_LFEx6 Bmdhcif

    - Measured Demand Quantity over CAISO Control Area – Load-Following Net Settlement and Exceptions #6 (by BA and Settlement Interval)

Where

BASettlementIntervalMeasuredDemandControlAreaQty_LFEx6 Bmdhcif = 
uT'I'M'W'VL'BAUDCSettlementIntervalMeasuredDemandControlAreaQty_LFEx6 BuT’I’M’W’VL’mdhcif

    - Measured Demand Quantity over UDC Area – Load-Following Net Settlement and Exceptions #6 (by Settlement Interval)

UDCTotalSettlementIntervalMeasuredDemandControlAreaQty_LFEx6 uT’I’M’W’VL’mdhcif = BBAUDCSettlementIntervalMeasuredDemandControlAreaQty_LFEx6 BuT’I’M’W’VL’mdhcif

    - Measured Demand Quantity over UDC Area – Load-Following Net Settlement and Exceptions #6 (by BA and Trading Hour)

Where

BAUDCHourlyMeasuredDemandControlAreaQty_LFEx6 BuT’I’M’W’VL’mdh = cf∈cBAUDCSettlementIntervalMeasuredDemandControlAreaQty_LFEx6 BuT’I’M’W’VL’mdhcif

    - Measured Demand Quantity over UDC Area – Load-Following Net Settlement and Exceptions #6 (by BA and Settlement Interval)

Where

BAUDCSettlementIntervalMeasuredDemandControlAreaQty_LFEx6 BuT’I’M’W’VL’mdhcif = BASettlementIntervalUDCTotalMeteredCAISODemandQuantity_LFEx6_MDOverCA BuT’I’M’W’VL’mdhcif 
+ BASettlementIntervalUDCTotalNetMSSMeasuredDemandQty_LFEx6_MDOverCA BuT’I’M’W’VL’mdhcif 
+ BASettlementIntervalUDCExportQuantity_Ex6_MDOverCA BuT’I’M’W’VL’mdhcif

      - Where

BASettlementIntervalUDCTotalMeteredCAISODemandQuantity_LFEx6_MDOverCA BuT’I’M’W’VL’mdhcif = rtQ'AA'R'pPQd'Nz'vHn'BAResSettlementIntervalMeteredCAISODemandQuantity BrtuT’I’Q’M’AA’R’pPW’Qd’Nz’VvHn’L’mdhcif

        - Where

(

T’ = ‘UDC’

        - Or Where

T’ = ‘MSS’ And I’ = ’GROSS’ And L’ = ‘NO’

)

        - And Where

MeasuredDemandControlAreaExceptions6Flag Brt <> True

        - And Where

BAMeasuredDemandBCRExceptionsFlag B  <> True

      - And Where

BASettlementIntervalUDCTotalNetMSSMeasuredDemandQty_LFEx6_MDOverCA BuT’I’M’W’VL’mdhcif = 
AA'BASettlementIntervalNetMSSMeasuredDemandQuantity BuT’I’M’AA’W’VL’mdhcif

        - Where

T’ = ‘MSS’

        - And Where

I’ = ‘NET’ Or L’ = ‘YES’

        - And Where

BAMeasuredDemandBCRExceptionsFlag B  <> True

      - And Where

BASettlementIntervalUDCExportQuantity_Ex6_MDOverCA BuT’I’M’W’VL’mdhcif = 
rtE Q'AA'F'R'pPQS'd'Nz'OvHn' (SettlementIntervalDeemedDeliveredInterchangeEnergyQuantity BrtEuT’I’Q’M’AA’F’R’pPW’QS’d’Nz’oVvHn’L’mdhcif + Op_Agreement_Export_Loss_Allocation_Quantity BrtEuT’I’Q’M’AA’F’R’pPW’QS’d’Nz’oVvHn’L’mdhcif)

        - Where

t = ‘ETIE’ and Q’ = ‘CISO’

        - And Where

E In ( ‘FIRM’, ‘NFRM’, ‘WHEEL’, ‘DYN’, ‘UCTG’)

        - And Where

(

T’ = ‘UDC’

        - Or Where

T’ = ‘MSS’ And I’ = ’GROSS’ And L’ = ‘NO’

)

        - And Where

MeasuredDemandControlAreaExceptions6Flag Brt <> True

        - And Where

BAMeasuredDemandBCRExceptionsFlag B  <> True

-- BCR-Related Measured Demand for Load-Following MSS Entities–-

    - BCR Tier 1 Measured Demand Quantity Excluding Transmission Rights over CAISO Control Area – Load-Following Net Settlement and Exceptions #6 (by BA and Trading Hour)

BAHourlyMeasuredDemandMinusRightsQuantity_NON_LF_EX_RTM_BCR Bmdh  = 
Min (0, (BAHourlyUDCOrMSSMeasuredDemandQuantity_NON_LF_EX_RTM_BCR Bmdh  - BAHourlyTORContractDemandQuantity_EX_BCR Bmdh))

      - Where

BAHourlyTORContractDemandQuantity_EX_BCR Bmdh   = rtF'S'cf∈c
BASettlementIntervalResTOR_MDControlAreaQty_BCR BrtF’S’mdhcif

        - Where

MeasuredDemandControlAreaExceptions6Flag Brt <> True

        - And Where

BAMeasuredDemandBCRExceptionsFlag B <> True

      - Where

BAHourlyUDCOrMSSMeasuredDemandQuantity_NON_LF_EX_RTM_BCR Bmdh  = uT'I'M'VL'cf∈c(BASettlementIntervalMSSNetMeasuredDemandQuantity_NON_LF_EX_RTM_BCR BuT’I’M’W’VL’mdhcif  + BASettlementIntervalUDCTotalMeteredCAISODemandQuantity_LFEx6_MDOverCA BuT’I’M’W’VL’mdhcif  + BASettlementIntervalUDCExportQuantity_Ex6_MDOverCA BuT’I’M’W’VL’mdhcif )

      - And Where

BASettlementIntervalMSSNetMeasuredDemandQuantity_NON_LF_EX_RTM_BCR BuT’I’M’W’VL’mdhcif = 
AA'BASettlementIntervalNetMSSMeasuredDemandQuantity BuT’I’M’AA’W’VL’mdhcif

        - Where

T’ = ‘MSS’

        - And Where

I’ = ‘NET’ And L’ = ‘NO’

        - And Where

BAMeasuredDemandBCRExceptionsFlag B  <> True

-- CC6774 – Related Measured Demand for UDC and All MSS Entities independent of Gross/Net Settlement Option, Excluding Transmission Loss Energy for Demand having Contract Rights–-

    - Real-time Congestion Offset-associated Measured Demand Quantity Excluding Energy having Transmission Rights, calculated over the CAISO Control Area (CAISO Total by Trading Hour and applying Exceptions #7)

CAISOHourlyMeasuredDemandMinusBalancedRightsQuantity_EX_RTM_CONGOFF mdh = BBAHourlyMeasuredDemandMinusBalancedRightsQuantity_EX_RTM_CONGOFF Bmdh

    - Real-time Congestion Offset-associated Measured Demand Quantity Excluding Energy having Transmission Rights, calculated over the CAISO Control Area (by BA and Trading Hour and applying Exceptions #7)

BAHourlyMeasuredDemandMinusBalancedRightsQuantity_EX_RTM_CONGOFF Bmdh = cf∈cBASettlementIntervalMeasuredDemandMinusBalancedRightsQuantity_EX_RTM_CONGOFF Bmdhcif

    - Real-time Congestion Offset-associated Measured Demand Quantity Excluding Energy having Transmission Rights, calculated over the CAISO Control Area (by BA and Settlement Interval and applying Exceptions #7)

Where

BASettlementIntervalMeasuredDemandMinusBalancedRightsQuantity_EX_RTM_CONGOFF Bmdhcif = 
(BASettlementIntervalEntityMeasuredDemandMinusBalancedRightsQuantity_EX_RTM_CONGOFF BuT’I’M’W’VL’mdhcif)

      - Where

BAMeasuredDemandCONGOFFExceptions7Flag B <> True

      - And Where

BASettlementIntervalEntityMeasuredDemandMinusBalancedRightsQuantity_EX_RTM_CONGOFF BuT’I’M’W’VL’mdhcif = (BASettlementIntervalMSSNetMeasuredDemandMinusBalancedRightsQuantity_EX_CONGOFF BuT’I’M’W’VL’mdhcif  + BASettlementIntervalUDCTotalMeteredCAISODemandQuantity_EX_CONGOFF BuT’I’M’W’VL’mdhcif  + BASettlementIntervalUDCExportQuantity_EX_CONGOFF BuT’I’M’W’VL’mdhcif–- BASettlementIntervalNonNetBalancedRightsDemandQuantity_EX_CONGOFF BuT’I’M’W’VL’mdhcif)

        - Where

BASettlementIntervalMSSNetMeasuredDemandMinusBalancedRightsQuantity_EX_CONGOFF BuT’I’M’W’VL’mdhcif = 
BASettlementIntervalNetMSSDemandMinusBalancedRightsNonExportTotalQuantity_EX_CONGOFF BuT’I’M’W’VL’mdhcif + BASettlementIntervalExportsMinusBalancedRightsExportTotalQuantity_EX_CONGOFF BuT’I’M’W’VL’mdhcif

          - Where

T’ = ‘MSS’

          - And Where

BASettlementIntervalNetMSSDemandMinusBalancedRightsNonExportTotalQuantity_EX_CONGOFF BuT’I’M’W’VL’mdhcif =

MIN(0, BASettlementIntervalNetMSSDemandTotalQuantity BuT’I’M’W’VL’mdhcif–- BASettlementIntervalEntityExceptionsNonExportTotalQuantity_EX_CONGOFF BuT’I’M’W’VL’mdhcif–- BASettlementIntervalEntityBalancedRightsNonExportTotalQuantity_EX_CONGOFF BuT’I’M’W’VL’mdhcif)

            - Where

BASettlementIntervalNetMSSDemandTotalQuantity BuT’I’M’W’VL’mdhcif = 
AA' BASettlementIntervalMSSDemandQuantity_MSSNetting BuT’I’M’AA’W’VL’mdhcif

            - And Where

BASettlementIntervalEntityExceptionsNonExportTotalQuantity_EX_CONGOFF BuT’I’M’W’VL’mdhcif = tAA'BASettlementIntervalEntityExceptionsQuantity_EX_CONGOFF BtuT’I’M’AA’W’VL’mdhcif

              - Where

t <> ‘ETIE’

              - And Where

BASettlementIntervalEntityExceptionsQuantity_EX_CONGOFF BtuT’I’M’AA’W’VL’mdhcif =

MeasuredDemandControlAreaCONGOFFExceptions7Flag Brt * (BAResSettlementIntervalMeteredCAISODemandQuantity BrtuT’I’Q’M’AA’R’pPW’Qd’Nz’VvHn’L’mdhcif+ SettlementIntervalDeemedDeliveredInterchangeEnergyExportTotalQuantity BrtuT’I’Q’M’AA’R’pPW’Qd’Nz’VvHn’L’mdhcif)

                - Where

SettlementIntervalDeemedDeliveredInterchangeEnergyExportTotalQuantity BrtuT’I’Q’M’AA’R’pPW’Qd’Nz’VvHn’L’mdhcif = (SettlementIntervalDeemedDeliveredInterchangeEnergyQuantity BrtEuT’I’Q’M’AA’F’R’pPW’QS’d’Nz’oVvHn’L’mdhcif + Op_Agreement_Export_Loss_Allocation_Quantity BrtEuT’I’Q’M’AA’F’R’pPW’QS’d’Nz’oVvHn’L’mdhcif)

- Where

E In ( ‘FIRM’, ‘NFRM’, ‘WHEEL’, ‘DYN’, ‘UCTG’)

- And Where

t = ‘ETIE’ and Q’ = ‘CISO’

            - And Where

BASettlementIntervalEntityBalancedRightsNonExportTotalQuantity_EX_CONGOFF BuT’I’M’W’VL’mdhcif = tAA'z'BASettlementIntervalEntityBalancedContractTypeDemandQuantity_EX_CONGOFF BtuT’I’M’AA’W’z’VL’mdhcif

              - Where

t <> ‘ETIE’

              - And Where

BASettlementIntervalEntityBalancedContractTypeDemandQuantity_EX_CONGOFF BtuT’I’M’AA’W’z’VL’mdhcif = BASettlementIntervalEntityBalancedContractTypeDemandQuantity BtuT’I’M’AA’W’z’VL’mdhcif–- BASettlementIntervalEntityBalancedContractTypeExceptionsDemandQuantity_EX_CONGOFF BtuT’I’M’AA’W’z’VL’mdhcif

                - Where

BASettlementIntervalEntityBalancedContractTypeDemandQuantity BtuT’I’M’AA’W’z’VL’mdhcif = 
rQ'R'pPQd'vHn'(SettlementIntervalResourceContractMD Brtz’mdhcif + BASettlementIntervalResourceContractTypeMDZero BrtuT’I’Q’M’AA’R’pPW’Qd’z’VvHn’L’mdhcif )

- Where

BASettlementIntervalResourceContractTypeMDZero BrtuT’I’Q’M’AA’R’pPW’Qd’z’VvHn’L’mdhcif = N0 * (BAResSettlementIntervalMeteredCAISODemandQuantity BrtuT’I’Q’M’AA’R’pPW’Qd’Nz’VvHn’L’mdhcif+ SettlementIntervalDeemedDeliveredInterchangeEnergyExportTotalQuantity BrtuT’I’Q’M’AA’R’pPW’Qd’Nz’VvHn’L’mdhcif)

                - And Where

BASettlementIntervalEntityBalancedContractTypeExceptionsDemandQuantity_EX_CONGOFF BtuT’I’M’AA’W’z’VL’mdhcif = rQ'R'pPQd'vHn'MeasuredDemandControlAreaCONGOFFExceptions7Flag Brt * (SettlementIntervalResourceContractMD Brtz’mdhcif + BASettlementIntervalResourceContractTypeMDZero BrtuT’I’Q’M’AA’R’pPW’Qd’z’VvHn’L’mdhcif)

          - And Where

BASettlementIntervalExportsMinusBalancedRightsExportTotalQuantity_EX_CONGOFF BuT’I’M’W’VL’mdhcif = 
AA'(BASettlementIntervalEntityExportQuantity BuT’I’M’AA’W’VL’mdhcif–- BASettlementIntervalEntityExceptionsExportQuantity_EX_CONGOFF BuT’I’M’AA’W’VL’mdhcif–- BASettlementIntervalEntityBalancedRightsExportQuantity_EX_CONGOFF BuT’I’M’AA’W’VL’mdhcif)

            - Where

BASettlementIntervalEntityExportQuantity BuT’I’M’AA’W’VL’mdhcif = BASettlementIntervalEntityQuantity BtuT’I’M’AA’W’VL’mdhcif

              - Where

t = ‘ETIE’

              - And Where

BASettlementIntervalEntityQuantity BtuT’I’M’AA’W’VL’mdhcif = (BAResSettlementIntervalMeteredCAISODemandQuantity BrtuT’I’Q’M’AA’R’pPW’Qd’Nz’VvHn’L’mdhcif+ SettlementIntervalDeemedDeliveredInterchangeEnergyExportTotalQuantity BrtuT’I’Q’M’AA’R’pPW’Qd’Nz’VvHn’L’mdhcif)

            - And Where

BASettlementIntervalEntityExceptionsExportQuantity_EX_CONGOFF BuT’I’M’AA’W’VL’mdhcif = tBASettlementIntervalEntityExceptionsQuantity_EX_CONGOFF BtuT’I’M’AA’W’VL’mdhcif

              - Where

t = ‘ETIE’

            - And Where

BASettlementIntervalEntityBalancedRightsExportQuantity_EX_CONGOFF BuT’I’M’AA’W’VL’mdhcif = BASettlementIntervalEntityBalancedContractTypeDemandQuantity_EX_CONGOFF BtuT’I’M’AA’W’z’VL’mdhcif)

              - Where

t = ‘ETIE’

        - And Where

BASettlementIntervalUDCTotalMeteredCAISODemandQuantity_EX_CONGOFF BuT’I’M’W’VL’mdhcif =

BASettlementIntervalEntityNonExportTotalDemandQuantity BuT’I’M’W’VL’mdhcif–- BASettlementIntervalEntityExceptionsNonExportTotalQuantity_EX_CONGOFF BuT’I’M’W’VL’mdhcif

          - Where

T’ = ‘UDC’

          - And Where

BASettlementIntervalEntityNonExportTotalDemandQuantity BuT’I’M’W’VL’mdhcif = tAA'BASettlementIntervalEntityQuantity BtuT’I’M’AA’W’VL’mdhcif

            - Where

t <> ‘ETIE’

        - And Where

BASettlementIntervalUDCExportQuantity_EX_CONGOFF BuT’I’M’W’VL’mdhcif = (BASettlementIntervalEntityExportQuantity BuT’I’M’AA’W’VL’mdhcif–- BASettlementIntervalEntityExceptionsExportQuantity_EX_CONGOFF BuT’I’M’AA’W’VL’mdhcif)

          - Where

T’ = ‘UDC’

        - And Where

BASettlementIntervalNonNetBalancedRightsDemandQuantity_EX_CONGOFF BuT’I’M’W’VL’mdhcif =

BASettlementIntervalEntityBalancedRightsNonExportTotalQuantity_EX_CONGOFF BuT’I’M’W’VL’mdhcif + BASettlementIntervalEntityBalancedRightsExportTotalQuantity_EX_CONGOFF BuT’I’M’W’VL’mdhcif

          - Where

T’ = ‘UDC’

          - And Where

BASettlementIntervalEntityBalancedRightsExportTotalQuantity_EX_CONGOFF BuT’I’M’W’VL’mdhcif = AA'(BASettlementIntervalEntityBalancedRightsExportQuantity_EX_CONGOFF BuT’I’M’AA’W’VL’mdhcif)

-- CC6477 – Related Measured Demand for UDC, Gross-Settled Non-Load-Following MSS, Net-Settled MSS and Load-Following MSS Entities, Excluding Transmission Loss Energy for Demand having TOR Rights–-

    - Real-time Imbalance Energy Offset–- associated Measured Demand Quantity Excluding Transmission Loss Energy for Demand having TOR Rights, calculated over the CAISO Control Area (CAISO Total by Settlement Interval and applying Exceptions #8)

CAISOSettlementIntervalMeasuredDemandMinusBalancedTORLossQuantity_EX_RTM_IMBOFF mdhcif = BBASettlementIntervalMeasuredDemandMinusBalancedTORLossQuantity_EX_RTM_IMBOFF Bmdhcif

    - Real-time Congestion Offset-associated Measured Demand Quantity Excluding Transmission Loss Energy for Demand having TOR Rights, calculated over the CAISO Control Area (by BA and Settlement Interval and applying Exceptions #8)

Where

BASettlementIntervalMeasuredDemandMinusBalancedTORLossQuantity_EX_RTM_IMBOFF Bmdhcif = 
(BASettlementIntervalEntityMeasuredDemandMinusBalancedTORLossQuantity_EX_RTM_IMBOFF BuT’I’M’W’VL’mdhcif)

      - Where

BAMeasuredDemandIMBOFFExceptions8Flag B <> True

      - And Where

BASettlementIntervalEntityMeasuredDemandMinusBalancedTORLossQuantity_EX_RTM_IMBOFF BuT’I’M’W’VL’mdhcif = (BASettlementIntervalMSSNetMeasuredDemandMinusBalancedTORLossQuantity_EX_IMBOFF BuT’I’M’W’VL’mdhcif  + BASettlementIntervalUDCTotalMeteredCAISODemandQuantity_EX_IMBOFF BuT’I’M’W’VL’mdhcif  + BASettlementIntervalUDCExportQuantity_EX_IMBOFF BuT’I’M’W’VL’mdhcif–- BASettlementIntervalNonNetBalancedTORDemandLossQuantity_EX_IMBOFF BuT’I’M’W’VL’mdhcif)

        - Where

BASettlementIntervalMSSNetMeasuredDemandMinusBalancedTORLossQuantity_EX_IMBOFF BuT’I’M’W’VL’mdhcif = 
BASettlementIntervalNetMSSDemandMinusBalancedTORNonExportLossTotalQuantity_EX_IMBOFF BuT’I’M’W’VL’mdhcif + BASettlementIntervalExportsMinusBalancedTORExportLossTotalQuantity_EX_IMBOFF BuT’I’M’W’VL’mdhcif

          - Where

T’ = ‘MSS’

          - And Where

(

I’ = ‘NET’

          - Or Where

L’ = ‘YES’

)

          - And Where

BASettlementIntervalNetMSSDemandMinusBalancedTORNonExportLossTotalQuantity_EX_IMBOFF BuT’I’M’W’VL’mdhcif =

MIN(0, BASettlementIntervalNetMSSDemandTotalQuantity BuT’I’M’W’VL’mdhcif–- BASettlementIntervalEntityExceptionsNonExportTotalQuantity_EX_IMBOFF BuT’I’M’W’VL’mdhcif–- BASettlementIntervalEntityBalancedTORNonExportLossTotalQuantity_EX_IMBOFF BuT’I’M’W’VL’mdhcif)

            - Where

BASettlementIntervalEntityExceptionsNonExportTotalQuantity_EX_IMBOFF BuT’I’M’W’VL’mdhcif = tAA'BASettlementIntervalEntityExceptionsQuantity_EX_IMBOFF BtuT’I’M’AA’W’VL’mdhcif

              - Where

t <> ‘ETIE’

              - And Where

BASettlementIntervalEntityExceptionsQuantity_EX_IMBOFF BtuT’I’M’AA’W’VL’mdhcif =

MeasuredDemandControlAreaIMBOFFExceptions8Flag Brt * (BAResSettlementIntervalMeteredCAISODemandQuantity BrtuT’I’Q’M’AA’R’pPW’Qd’Nz’VvHn’L’mdhcif+ SettlementIntervalDeemedDeliveredInterchangeEnergyExportTotalQuantity BrtuT’I’Q’M’AA’R’pPW’Qd’Nz’VvHn’L’mdhcif)

            - And Where

BASettlementIntervalEntityBalancedTORNonExportLossTotalQuantity_EX_IMBOFF BuT’I’M’W’VL’mdhcif = tAA'z'BASettlementIntervalEntityBalancedContractTypeDemandLossQuantity_EX_IMBOFF BtuT’I’M’AA’W’z’VL’mdhcif

              - Where

t <> ‘ETIE’

              - And Where

z’ = ‘TOR’

              - And Where

BASettlementIntervalEntityBalancedContractTypeDemandLossQuantity_EX_IMBOFF BtuT’I’M’AA’W’z’VL’mdhcif = BASettlementIntervalEntityBalancedContractTypeDemandLossQuantity BtuT’I’M’AA’W’z’VL’mdhcif–- BASettlementIntervalEntityBalancedContractTypeExceptionsDemandLossQuantity_EX_IMBOFF BtuT’I’M’AA’W’z’VL’mdhcif

                - Where

BASettlementIntervalEntityBalancedContractTypeDemandLossQuantity BtuT’I’M’AA’W’z’VL’mdhcif =
rQ'R'pPQd'vHn' (BASettlementIntervalResourceEnergyLossCreditEligibleCRNDemandQuantity Brtz’mdhcif + BASettlementIntervalResourceContractTypeMDZero BrtuT’I’Q’M’AA’R’pPW’Qd’z’VvHn’L’mdhcif)

                - And Where

BASettlementIntervalEntityBalancedContractTypeExceptionsDemandLossQuantity_EX_IMBOFF BtuT’I’M’AA’W’z’VL’mdhcif = rQ'R'pPQd'vHn' MeasuredDemandControlAreaIMBOFFExceptions8Flag Brt * (BASettlementIntervalResourceEnergyLossCreditEligibleCRNDemandQuantity Brtz’mdhcif + BASettlementIntervalResourceContractTypeMDZero BrtuT’I’Q’M’AA’R’pPW’Qd’z’VvHn’L’mdhcif)

          - And Where

BASettlementIntervalExportsMinusBalancedTORExportLossTotalQuantity_EX_IMBOFF BuT’I’M’W’VL’mdhcif = 
AA'(BASettlementIntervalEntityExportQuantity BuT’I’M’AA’W’VL’mdhcif–- BASettlementIntervalEntityExceptionsExportQuantity_EX_IMBOFF BuT’I’M’AA’W’VL’mdhcif–- BASettlementIntervalEntityBalancedTORExportLossQuantity_EX_IMBOFF BuT’I’M’AA’W’VL’mdhcif)

            - And Where

BASettlementIntervalEntityExceptionsExportQuantity_EX_IMBOFF BuT’I’M’AA’W’VL’mdhcif = tBASettlementIntervalEntityExceptionsQuantity_EX_IMBOFF BtuT’I’M’AA’W’VL’mdhcif

              - Where

t = ‘ETIE’

            - And Where

BASettlementIntervalEntityBalancedTORExportLossQuantity_EX_IMBOFF BuT’I’M’AA’W’VL’mdhcif = z't(BASettlementIntervalEntityBalancedContractTypeDemandLossQuantity_EX_IMBOFF BtuT’I’M’AA’W’z’VL’mdhcif)

              - Where

z’ = ‘TOR’

              - And Where

t = ‘ETIE’

        - And Where

BASettlementIntervalUDCTotalMeteredCAISODemandQuantity_EX_IMBOFF BuT’I’M’W’VL’mdhcif =

(BASettlementIntervalEntityNonExportTotalDemandQuantity BuT’I’M’W’VL’mdhcif–- BASettlementIntervalEntityExceptionsNonExportTotalQuantity_EX_IMBOFF BuT’I’M’W’VL’mdhcif)

          - Where

(

T’ = ‘UDC’

          - Or Where

T’ = ‘MSS’ And I’ = ’GROSS’ And L’ = ‘NO’

)

        - And Where

BASettlementIntervalUDCExportQuantity_EX_IMBOFF BuT’I’M’W’VL’mdhcif = (BASettlementIntervalEntityExportQuantity BuT’I’M’AA’W’VL’mdhcif–- BASettlementIntervalEntityExceptionsExportQuantity_EX_IMBOFF BuT’I’M’AA’W’VL’mdhcif)

          - Where

(

T’ = ‘UDC’

          - Or Where

T’ = ‘MSS’ And I’ = ’GROSS’ And L’ = ‘NO’

)

        - And Where

BASettlementIntervalNonNetBalancedTORDemandLossQuantity_EX_IMBOFF BuT’I’M’W’VL’mdhcif =

BASettlementIntervalEntityBalancedTORNonExportLossTotalQuantity_EX_IMBOFF BuT’I’M’W’VL’mdhcif + BASettlementIntervalEntityBalancedTORExportLossTotalQuantity_EX_IMBOFF BuT’I’M’W’VL’mdhcif

          - Where

(

T’ = ‘UDC’

          - Or Where

T’ = ‘MSS’ And I’ = ’GROSS’ And L’ = ‘NO’

)

          - And Where

BASettlementIntervalEntityBalancedTORExportLossTotalQuantity_EX_IMBOFF BuT’I’M’W’VL’mdhcif = AA'BASettlementIntervalEntityBalancedTORExportLossQuantity_EX_IMBOFF BuT’I’M’AA’W’VL’mdhcif

-- CC6477 – Related Measured Demand for UDC, Gross-Settled Non-Load-Following MSS, Net-Settled MSS and Load-Following MSS Entities, Excluding Measured Demand Energy for Demand having TOR Rights–-

    - Real-time Imbalance Energy Offset–- associated Measured Demand Quantity Excluding Measured Demand Energy for Demand having TOR Rights, calculated over the CAISO Control Area (CAISO Total by Settlement Interval and applying Exceptions #8)

CAISOSettlementIntervalMeasuredDemandMinusBalancedTORDemandQuantity_EX_RTM_IMBOFF mdhcif = BBASettlementIntervalMeasuredDemandMinusBalancedTORDemandQuantity_EX_RTM_IMBOFF Bmdhcif

    - Real-time Congestion Offset-associated Measured Demand Quantity Excluding Transmission Measured Demand Energy for Demand having TOR Rights, calculated over the CAISO Control Area (by BA and Settlement Interval and applying Exceptions #8)

Where

BASettlementIntervalMeasuredDemandMinusBalancedTORDemandQuantity_EX_RTM_IMBOFF Bmdhcif = 
uT'I'M'VL'(BASettlementIntervalEntityMeasuredDemandMinusBalancedTORDemandQuantity_EX_RTM_IMBOFF BuT’I’M’W’VL’mdhcif)

      - Where

BAMeasuredDemandIMBOFFExceptions8Flag B <> True

      - And Where

BASettlementIntervalEntityMeasuredDemandMinusBalancedTORDemandQuantity_EX_RTM_IMBOFF BuT’I’M’W’VL’mdhcif= (BASettlementIntervalMSSNetMeasuredDemandMinusBalancedTORDemandQuantity_EX_IMBOFF BuT’I’M’W’VL’mdhcif  + BASettlementIntervalUDCTotalMeteredCAISODemandQuantity_EX_IMBOFF BuT’I’M’W’VL’mdhcif  + BASettlementIntervalUDCExportQuantity_EX_IMBOFF BuT’I’M’W’VL’mdhcif–- BASettlementIntervalNonNetBalancedTORDemandQuantity_EX_IMBOFF BuT’I’M’W’VL’mdhcif)

        - Where

BASettlementIntervalMSSNetMeasuredDemandMinusBalancedTORDemandQuantity_EX_IMBOFF BuT’I’M’W’VL’mdhcif = 
BASettlementIntervalNetMSSDemandMinusBalancedTORNonExportDemandTotalQuantity_EX_IMBOFF BuT’I’M’W’VL’mdhcif + BASettlementIntervalExportsMinusBalancedTORExportTotalQuantity_EX_IMBOFF BuT’I’M’W’VL’mdhcif

          - Where

T’ = ‘MSS’

          - And Where

(

I’ = ‘NET’

          - Or Where

L’ = ‘YES’

)

          - And Where

BASettlementIntervalNetMSSDemandMinusBalancedTORNonExportDemandTotalQuantity_EX_IMBOFF BuT’I’M’W’VL’mdhcif =

MIN(0, BASettlementIntervalNetMSSDemandTotalQuantity BuT’I’M’W’VL’mdhcif–- BASettlementIntervalEntityExceptionsNonExportTotalQuantity_EX_IMBOFF BuT’I’M’W’VL’mdhcif–- BASettlementIntervalEntityBalancedTORNonExportTotalQuantity_EX_IMBOFF BuT’I’M’W’VL’mdhcif)

            - Where

BASettlementIntervalEntityBalancedTORNonExportTotalQuantity_EX_IMBOFF BuT’I’M’W’VL’mdhcif = tAA'z'BASettlementIntervalEntityBalancedContractTypeDemandQuantity_EX_IMBOFF BtuT’I’M’AA’W’z’VL’mdhcif

              - Where

t <> ‘ETIE’

              - And Where

z’ = ‘TOR’

              - And Where

BASettlementIntervalEntityBalancedContractTypeDemandQuantity_EX_IMBOFF BtuT’I’M’AA’W’z’VL’mdhcif = BASettlementIntervalEntityBalancedContractTypeDemandQuantity BtuT’I’M’AA’W’z’VL’mdhcif–- BASettlementIntervalEntityBalancedContractTypeExceptionsDemandQuantity_EX_IMBOFF BtuT’I’M’AA’W’z’VL’mdhcif

                - Where

BASettlementIntervalEntityBalancedContractTypeExceptionsDemandQuantity_EX_IMBOFF BtuT’I’M’AA’W’z’VL’mdhcif = rQ'R'pPQd'vHn' MeasuredDemandControlAreaIMBOFFExceptions8Flag Brt * (SettlementIntervalResourceContractMD Brtz’mdhcif + BASettlementIntervalResourceContractTypeMDZero BrtuT’I’Q’M’AA’R’pPW’Qd’z’VvHn’L’mdhcif)

          - And Where

BASettlementIntervalExportsMinusBalancedTORExportTotalQuantity_EX_IMBOFF BuT’I’M’W’VL’mdhcif = 
AA'(BASettlementIntervalEntityExportQuantity BuT’I’M’AA’W’VL’mdhcif–- BASettlementIntervalEntityExceptionsExportQuantity_EX_IMBOFF BuT’I’M’AA’W’VL’mdhcif–- BASettlementIntervalEntityBalancedTORExportQuantity_EX_IMBOFF BuT’I’M’AA’W’VL’mdhcif)

            - Where

BASettlementIntervalEntityBalancedTORExportQuantity_EX_IMBOFF BuT’I’M’AA’W’VL’mdhcif = z't(BASettlementIntervalEntityBalancedContractTypeDemandQuantity_EX_IMBOFF BtuT’I’M’AA’W’z’VL’mdhcif)

              - Where

z’ = ‘TOR’

              - And Where

t = ‘ETIE’

        - And Where

BASettlementIntervalNonNetBalancedTORDemandQuantity_EX_IMBOFF BuT’I’M’W’VL’mdhcif =

BASettlementIntervalEntityBalancedTORNonExportTotalQuantity_EX_IMBOFF BuT’I’M’W’VL’mdhcif + BASettlementIntervalEntityBalancedTORExportTotalQuantity_EX_IMBOFF BuT’I’M’W’VL’mdhcif

          - Where

(

T’ = ‘UDC’

          - Or Where

T’ = ‘MSS’ And I’ = ’GROSS’ And L’ = ‘NO’

)

          - And Where

BASettlementIntervalEntityBalancedTORExportTotalQuantity_EX_IMBOFF BuT’I’M’W’VL’mdhcif = AA'BASettlementIntervalEntityBalancedTORExportQuantity_EX_IMBOFF BuT’I’M’AA’W’VL’mdhcif

Pre-calculation Measured Demand over TAC Area – CPM  
– Related Metered Load by TAC Area, excluding Load having TOR Rights –

    - BAResource10MMeteredTACAreaDemandQuantity Brtvmdhi =

uT'I'Q'M'AA'R'pPW'Qd'Nz'VHn'L'f∈i BAResSettlementIntervalMeteredCAISODemandQuantity BrtuT’I’Q’M’AA’R’pPW’Qd’Nz’VvHn’L’mdhcif

    - BAResource10MTORContractMD Brtmdhi =

z'f∈iSettlementIntervalResourceContractMD Brtz’mdhcif

      - Where

z’ = ‘TOR’

CC 6457 – Related Measured Demand, excluding Load or Exports having TOR Rights –

    - Measured Demand Quantity Minus Balanced TOR Rights over CAISO Control Area – Exceptions #9 (by Trading Hour)

CAISOTotalHourlyMeasuredDemandMinusBalancedTOR_DeclinedHASPBidsQty mdh = 
B BAHourlyMeasuredDemandMinusBalancedTOR_DeclinedHASPBidsQty Bmdh

    - Measured Demand Quantity Minus Balanced TOR Rights over CAISO Control Area – Exceptions #9 (by BA and Trading Hour)

BAHourlyMeasuredDemandMinusBalancedTOR_DeclinedHASPBidsQty Bmdh = MIN (0, BAHourlyMeasuredDemand_DeclinedHASPBidsQty Bmdh–- BAHourlyBalancedTOR_DeclinedHASPBidsQty Bmdh)

    - Measured Demand Quantity over CAISO Control Area – Exceptions #9 (by BA and Trading Hour)

BAHourlyMeasuredDemand_DeclinedHASPBidsQty Bmdh =

uT'I'M'W'VL'cf∈cBAUDCSettlementIntervalMeasuredDemandControlAreaDeclinedHASPBidsQty BuT’I’M’W’VL’mdhcif

    - Measured Demand Quantity over UDC Area – Exceptions #9 (by BA and Settlement Interval)

Where

BAUDCSettlementIntervalMeasuredDemandControlAreaDeclinedHASPBidsQty BuT’I’M’W’VL’mdhcif =

BASettlementIntervalUDCTotalMeteredCAISODemandQuantity_Ex9_MDOverCA BuT’I’M’W’VL’mdhcif 
+ BASettlementIntervalUDCTotalNetMSSMeasuredDemandQty_Ex9_MDOverCA BuT’I’M’W’VL’mdhcif 
+ BASettlementIntervalUDCExportQuantity_Ex9_MDOverCA BuT’I’M’W’VL’mdhcif

      - Where

BASettlementIntervalUDCTotalMeteredCAISODemandQuantity_Ex9_MDOverCA BuT’I’M’W’VL’mdhcif = rtQ'AA'R'pPQd'Nz'vHn'BAResSettlementIntervalMeteredCAISODemandQuantity BrtuT’I’Q’M’AA’R’pPW’Qd’Nz’VvHn’L’mdhcif

        - Where

T’ = ‘UDC’

Or

T’ = ‘MSS’ And I’ = ’GROSS’

        - And Where

BAResourceMeasuredDemandDeclinedHASPBidsExceptions9Flag Brt <> True

        - And Where

BAMeasuredDemandDeclinedHASPBids9Flag B <> True

      - And Where

BASettlementIntervalUDCTotalNetMSSMeasuredDemandQty_Ex9_MDOverCA BuT’I’M’W’VL’mdhcif = 
AA'BASettlementIntervalNetMSSMeasuredDemandQuantity BuT’I’M’AA’W’VL’mdhcif

        - Where

T’ = ‘MSS’ And I’ = ‘NET’

        - And Where

BAMeasuredDemandDeclinedHASPBids9Flag B <> True

      - And Where

BASettlementIntervalUDCExportQuantity_Ex9_MDOverCA BuT’I’M’W’VL’mdhcif = 
rtE Q'AA'F'R'pPQS'd'Nz'OvHn' (SettlementIntervalDeemedDeliveredInterchangeEnergyQuantity BrtEuT’I’Q’M’AA’F’R’pPW’QS’d’Nz’oVvHn’L’mdhcif + Op_Agreement_Export_Loss_Allocation_Quantity BrtEuT’I’Q’M’AA’F’R’pPW’QS’d’Nz’oVvHn’L’mdhcif)

        - Where

t = ‘ETIE’ and Q’ = ‘CISO’

And

E In ( ‘FIRM’, ‘NFRM’, ‘WHEEL’, ‘DYN’, ‘UCTG’)

And

T’ = ‘UDC’ Or T’ = ‘MSS’ And I’ = ’GROSS’

        - And Where

BAResourceMeasuredDemandDeclinedHASPBidsExceptions9Flag Brt <> True

        - And Where

BAMeasuredDemandDeclinedHASPBids9Flag B <> True

    - Balanced TOR not Subject to Declined HASP Bids Exceptions (by BA and Settlement Interval)

Where

BAHourlyBalancedTOR_DeclinedHASPBidsQty Bmdh =

rtz'cf∈cSettlementIntervalResourceContractMD Brtz’mdhcif

      - Where

z’ = ‘TOR’

      - And Where

BAResourceMeasuredDemandDeclinedHASPBidsExceptions9Flag Brt <> True

      - And Where

BAMeasuredDemandDeclinedHASPBids9Flag B <> True

## Outputs

| Output Req ID | Name | Description |
| --- | --- | --- |
| 1 | In addition to any outputs listed below, all external inputs shall be included as outputs. | All inputs. |
| 2 | CAISOTotalHourlyMeasuredDemandControlAreaQty mdh | Total Measured Demand quantity calculated over the CAISO Control Area for a Trading Hour. |
| 3 | CAISOTotalSettlementIntervalMeasuredDemandControlAreaQty mdhcif | Total Measured Demand quantity calculated over the CAISO Control Area for a settlement interval. |
| 4 | BAHourlyMeasuredDemandControlAreaQty Bmdh | Hourly Measured Demand quantitycalculated over the CAISO Control Area . |
| 5 | BASettlementIntervalMeasuredDemandControlAreaQty Bmdhcif | Measured Demand quantity calculated over the CAISO Control Area in for a settlement interval. |
| 6 | UDCTotalSettlementIntervalMeasuredDemandControlAreaQty uT’I’M’W’VL’mdhcif | UDCTotal Measured Demand quantity calculated over the CAISO Control Area for a settlement interval. |
| 7 | BAUDCHourlyMeasuredDemandControlAreaQty BuT’I’M’W’VL’mdh | Hourly UDC Measured Demand quantity calculated for a Business Associate. |
| 8 | BAUDCSettlementIntervalMeasuredDemandControlAreaQty BuT’I’M’W’VL’mdhcif | UDC Measured Demand quantity calculated over the CAISO Control Area for a settlement interval. |
| 9 | BASettlementIntervalUDCTotalMeteredCAISODemandQuantity_MDOverCA BuT’I’M’W’VL’mdhcif | Sum of meter input values for all Loads and other non-export Demand resources, where the quantity is calculated for all UDC and gross-settled MSS entities for a settlement interval. The summed quantity is restricted to Energy from non-MSS entities or from gross-settled MSS entities. |
| 10 | BASettlementIntervalUDCNodalMeteredCAISODemandQuantity BuT’I’M’AA’W’VL’mdhcif | Sum of the individual meter readings for Loads and other non-export Demand resources of non-MSS entities and gross-settled MSS entities over the Aggregated Pricing, where the quantity is calculated for all UDC and MSS entities for a settlement interval. |
| 11 | BASettlementIntervalUDCTotalNetMSSMeasuredDemandQty_MDOverCA BuT’I’M’W’VL’mdhcif | Net MSS Measured Demand, where the quantity is calculated over all net-settled MSS entities for a settlement interval. |
| 12 | BASettlementIntervalUDCExportQuantity_MDOverCA BuT’I’M’W’VL’mdhcif | Sum of real-time deemed-delivered export quantities, including export losses settled under contractual operating agreements for a settlement interval. |
| 13 | HourlyNodalMeteredCAISODemandQuantity_MDOverCA AA’mdh | Hourly summation of meter input values over the Aggregated Pricing. The summed quantity includes the individual meter readings for Loads and other non-export Demand resources of non-MSS entities and gross-settled MSS entities, and also includes the calculated net MSS Demand value for each net-settled MSS entity. |
| 14 | SettlementIntervalNodalMeteredCAISODemandQuantity_MDOverCA uM’AA’mdhcif | Sum of meter input values over the Aggregated Pricing for a settlement interval. The summed quantity includes the individual meter readings for Loads and other non-export Demand resources of non-MSS entities and gross-settled MSS entities, and also includes the calculated net MSS Demand value for each net-settled MSS entity. |
| 14.1 | BAEntitySettlementIntervalNodalMeteredCAISODemandQuantity BuM’AA’mdhcif | Sum of meter input values over the Aggregated Pricing for a settlement interval. The summed quantity includes the individual meter readings for Loads and other non-export Demand resources of non-MSS entities and gross-settled MSS entities. Includes only entity component subtype with value ‘GL’ or ‘NPL’. |
| 15 | BAHourlyNodalMeteredCAISODemandQuantity_MDOverCA BAA’mdh | Hourly sum of meter input values over the Aggregated Pricing Node. The summed quantity includes the individual meter readings for Loads and other non-export Demand resources of non-MSS entities and gross-settled MSS entities, and also includes the calculated net MSS Demand value for each net-settled MSS entity. |
| 16 | BASettlementIntervalNodalMeteredCAISODemandQuantity_MDOverCA BAA’mdhcif | Sum over the Aggregated Pricing Node for asettlement interval of the individual meter readings for Loads and other non-export Demand resources of non-MSS entities and gross-settled MSS entities, and of the calculated net MSS Demand values for all net-settled MSS entities. |
| 17 | BASettlementIntervalNodalNetMSSDemandQuantity BAA’mdhcif | Sum of the calculated net MSS Demand values for all net-settled MSS entities over the Aggregated Pricing for a settlement interval. The summed quantity includes the calculated net MSS Demand value for each net-settled MSS entity. |
| 18 | BASettlementInterval_NodalMeteredCAISODemandQuantity BAA’mdhcif | Sum of the individual meter readings for Loads and other non-export Demand resources of non-MSS entities and gross-settled MSS entities over the Aggregated Pricing for a settlement interval. |
| 19 | CAISOTotalHourlyMeasuredDemandMinusRightsControlAreaQty mdh | Hourly total Measured Demand quantity minus Demand served through transmission rights of Business Associates, where the Measured Demand quantity represents the Demand of the CAISO Control Area. |
| 20 | CAISOTotalSettlementIntervalMeasuredDemandMinusRightsControlAreaQty mdhcif | Total Measured Demand quantity minus Demand served through transmission rights of Business Associates, where the Measured Demand quantity represents the Demand of the CAISO Control Area for a settlement interval. |
| 21 | BAHourlyMeasuredDemandMinusRightsControlAreaQty Bmdh | Houly Measured Demand quantity minus Demand served through transmission rights of Business Associates. |
| 22 | BASettlementIntervalMeasuredDemandMinusRightsControlAreaQty Bmdhcif | Measured Demand quantity minus Demand served through transmission rights of Business Associates, where the Measured Demand quantity represents the Demand served by a Business Associate over the CAISO Control Area for a settlement interval. |
| 23 | BASettlementIntervalRightsMDControlAreaQty Bmdhcif | Transmission rights of a Business Associate. The Business Associate is excluded from Measured Demand for a settlement interval. |
| 24 | CAISOTotalHourlyMeasuredDemandMinusRTRightsControlAreaQty mdh | Hourly total Measured Demand quantity minus Demand served through real-time transmission rights of Business Associates. |
| 25 | CAISOTotalSettlementIntervalMeasuredDemandMinusRTRightsControlAreaQty mdhcif | Total Measured Demand quantity minus Demand served through real-time transmission rights of Business Associates, where the Measured Demand quantity represents the Demand of the CAISO Control Area for a settlement interval. |
| 26 | BAHourlyMeasuredDemandMinusRTRightsControlAreaQty Bmdh | Hourly Measured Demand quantity minus Demand served through real-time transmission rights of Business Associates. |
| 27 | BASettlementIntervalMeasuredDemandMinusRTRightsControlAreaQty Bmdhcif | Measured Demand quantity minus Demand served through real-time transmission rights of Business Associates, where the Measured Demand quantity represents the Demand served by a Business Associate over the CAISO Control Area for a settlement interval. |
| 28 | BASettlementIntervalRTRights_MDControlAreaQty Bmdhcif | Real-time transmission rights of a Business Associate. The Business Associate is excluded from Measured Demand for a settlement interval. |
| 29 | CAISOTotalHourlyMeasuredDemandMinusRT_TORControlAreaQty mdh | Hourly total Measured Demand quantity minus Demand served through real-time TOR transmission rights of Business Associates, where the Measured Demand quantity represents the Demand of the CAISO Control Area. |
| 30 | CAISOTotalSettlementIntervalMeasuredDemandMinusRT_TORControlAreaQty mdhcif | Total Measured Demand quantity minus Demand served through real-time TOR transmission rights of Business Associates, where the Measured Demand quantity represents the Demand of the CAISO Control Area for a settlement interval. |
| 31 | BAHourlyMeasuredDemandMinusRT_TORControlAreaQty Bmdh | Hourly Measured Demand quantity minus Demand served through real-time TOR transmission rights of Business Associates, where the Measured Demand quantity represents the Demand served by Business Associate B over the CAISO Control. |
| 32 | BASettlementIntervalMeasuredDemandMinusRT_TORControlAreaQty Bmdhcif | Measured Demand quantity minus Demand served through real-time TOR transmission rights of Business Associate B, where the Measured Demand quantity represents the Demand served by Business Associate B over the CAISO Control Area for a settlement interval. |
| 33 | BASettlementIntervalRT_TOR_MDControlAreaQty Bmdhcif | Real-time TOR transmission rights of a Business Associate. Up to the value of the output quantity, the Business Associate is excluded from Measured Demand for a settlement interval. |
| 34 | CAISOTotalHourlyMeasuredDemandMinusRightsControlAreaQty_Ex1 mdh | Total Measured Demand quantity minus Demand served through transmission rights of Business Associates comprising Exceptions #5, where the Measured Demand quantity represents the Demand of the CAISO Control Area and excludes Energy provided by Business Associates and resources that comprise Exceptions #1. |
| 35 | CAISOTotal10MMeasuredDemandMinusRightsControlAreaQty_Ex1 mdhi | Total Measured Demand quantity minus Demand served through transmission rights of Business Associates comprising Exceptions #5, where the Measured Demand quantity represents the Demand of the CAISO Control Area over a 10-minute and excludes Energy provided by Business Associates and resources that comprise Exceptions #1. |
| 36 | BAHourlyMeasuredDemandMinusRightsControlAreaQty_Ex1 Bmdh | Measured Demand quantity minus Demand served through transmission rights of a Business Associate who belongs to Exceptions #5, where the Measured Demand quantity excludes Energy provided by Business Associates and resources that comprise Exceptions #1. |
| 37 | BA10MMeasuredDemandMinusRightsControlAreaQty_Ex1 Bmdhi | Measured Demand quantity minus Demand served through transmission rights of a Business Associate who belongs to Exceptions #5, where the Measured Demand quantity represents the Demand served by Business Associate B over the CAISO Control Area in a 10-minute interva and excludes Energy provided by Business Associates and resources that comprise Exceptions #1. |
| 38 | BASettlementIntervalRightsMDControlAreaQty_Ex5 Bmdhcif | Transmission rights Measured Demand Quantity for a Business Associate who belongs to Exceptions #5. The Business Associate is excluded from Measured Demand for a settlement interval. |
| 39 | CAISOTotalHourlyMeasuredDemandControlAreaQty_Ex1 mdh | Total Measured Demand quantity calculated over the CAISO Control Area with exclusion of Energy provided by Business Associates and resources that comprise Exceptions #1. |
| 40 | CAISOTotalSettlementIntervalMeasuredDemandControlAreaQty_Ex1 mdhcif | Total Measured Demand quantity calculated over the CAISO Control Area for a settlement interval, with exclusion of Energy provided by Business Associates and resources that comprise Exceptions #1. |
| 41 | BAHourlyMeasuredDemandControlAreaQty_Ex1 Bmdh | Hourly Measured Demand quantity calculated for a Business Associate over the CAISO Control Area with exclusion of Energy provided by Business Associates and resources that comprise Exceptions #1. |
| 42 | BASettlementIntervalMeasuredDemandControlAreaQty_Ex1 Bmdhcif | Measured Demand quantity calculated for a Business Associate over the CAISO Control for a settlement interval, with exclusion of Energy provided by Business Associates and resources that comprise Exceptions #1. |
| 43 | UDCTotalSettlementIntervalMeasuredDemandControlAreaQty_Ex1 uT’I’M’W’VL’mdhcif | UDCTotal Measured Demand quantity calculated over the CAISO Control Area for a settlement interval, with exclusion of Energy provided by Business Associates and resources that comprise Exceptions #1. |
| 44 | BAUDCHourlyMeasuredDemandControlAreaQty_Ex1 BuT’I’M’W’VL’mdh | UDC Measured Demand quantity calculated for a Business Associate associated with its UDC/MSS over the CAISO Control Area, with exclusion of Energy provided by Business Associates and resources that comprise Exceptions #1. |
| 45 | BAUDCSettlementIntervalMeasuredDemandControlAreaQty_Ex1 BuT’I’M’W’VL’mdhcif | UDC Measured Demand quantity calculated for a Business Associate and associated UDC/MSS over the CAISO Control Area for a settlement interval, with exclusion of Energy provided by Business Associates and resources that comprise Exceptions #1. |
| 46 | BASettlementIntervalUDCTotalMeteredCAISODemandQuantity_Ex1_MDOverCA BuT’I’M’W’VL’mdhcif | Sum of meter input values for all Loads and other non-export Demand resources where the quantity is calculated for all UDC and gross-settled MSS entities for a settlement interval, with exclusion of Energy provided by Business Associates and resources that comprise Exceptions #1. The summed quantity is restricted to Energy from non-MSS entities or from gross-settled MSS entities. |
| 47 | BASettlementIntervalUDCTotalNetMSSMeasuredDemandQty_Ex1_MDOverCA BuT’I’M’W’VL’mdhcif | Net MSS Measured Demand attributable to Business Associate B and UDC/MSS u of Entity Type T’ (in association with MSS subgroup M’ and MSS-related attributes MSS Gross/Net Energy Settlement Type I’, MSS Emissions Pay Flag W’, RUC Participation Flag V and Load-Following Selection Flag L’), where the quantity is calculated over all net-settled MSS entities for a settlement interval, with exclusion of Energy provided by Business Associates and resources that comprise Exceptions #1. |
| 48 | BASettlementIntervalUDCExportQuantity_Ex1_MDOverCA BuT’I’M’W’VL’mdhcif | Sum of real-time deemed-delivered export quantities, including export losses settled under contractual operating agreements, for a settlement interval, with exclusion of Energy provided by Business Associates and resources that comprise Exceptions #1. |
| 49 | <deleted> |  |
| 50 | <deleted> |  |
| 51 | <deleted> |  |
| 52 | <deleted> |  |
| 53 | <deleted> |  |
| 54 | UDCTotalSettlementIntervalGrossMeteredDemandControlAreaQty_Ex1 uT’I’M’W’VL’mdhcif | Total UDC Gross Metered Demand quantity calculated as the sum of the UDC Gross Metered Demand quantity over all Business Associates of a UDC/MSS for a settlement interval, with exclusion of Energy provided by Business Associates and resources that comprise Exceptions #1 as well as any Energy associated with TOR transmission rights. |
| 55 | BAUDCHourlyGrossMeteredDemandControlAreaQty_Ex1 BuT’I’M’W’VL’mdh | Hourly UDC Gross Metered Demand quantity calculated for a Business Associate associated with its UDC/MSS over the CAISO Control Area, with exclusion of Energy provided by Business Associates and resources that comprise Exceptions #1 as well as any Energy associated with TOR transmission rights. |
| 56 | BAUDCSettlementIntervalGrossMeteredDemandControlAreaQty_Ex1 BuT’I’M’W’VL’mdhcif | UDC Gross Metered Demand quantity calculated for a Business Associate associated with its UDC/MSS over the CAISO Control Area for a settlement interval, with exclusion of Energy provided by Business Associates and resources that comprise Exceptions #1 as well as any Energy associated with TOR transmission rights. |
| 57 | BASettlementIntervalUDCTotalNetMSSGrossLoadQty_Ex1_MDOverCA BuT’I’M’W’VL’mdhcif | Total MSS Gross Load attributable to a Business Associate where the quantity is calculated over all net-settled MSS entities for a settlement interval, with exclusion of Energy provided by Business Associates and resources that comprise Exceptions #1. |
| 58 | BASettlementIntervalMeteredDemandRightsMDControlAreaQty_UFE BuT’I’M’Vmdhcif | The quantity (in MWh) of gross metered Demand associated with a Business Associate for a settlement interval. |
| 59 | EntityBasicInfo BuT’I’M’Vmd | This CT output provides an entity’s basic attribute information. The output = 0 for all entities. For each UDC/MSS entity, the output presents the the entity type , any associated MSS subgroup values, and for each combination of entity and MSS subgroup, the corresponding values for the Aggregated Pricing Node. Additionally, for cases where T’ = ‘MSS’, u will be associated with the attributes gross/net settlement type I’, MSS Emissions Pay Flag W’, and RUC Participation Flag V that specify MSS operational or settlement selections. For a case where M’ is NULL (i.e., a MSS subgroup either does not exist for a MSS in a case where T’ = ‘MSS’ or does not apply to a UDC in a case where T’ = ‘UDC’), the other attributes of the output apply to u instead of M’, if they exist. The output applies to a Trading Day. |
| 60 | BASettlementIntervalTORMeteredDemand Bmdhcif | An output that presents the TOR transmission right-exempted Metered Demand quantity for a Business Associate and settlement interval over all Loads and other non-export Demand resources resources. |
| 61 | UDCTotalSettlementIntervalGrossMeasuredDemandControlAreaQty_Ex1 uT’I’M’W’VL’mdhcif | UDC Total Gross Measured Demand quantity calculated over the CAISO Control Area for UDC/MSS for a settlement interval, with exclusion of Energy provided by Business Associates and resources that comprise Exceptions #1. The output is similar to output UDCTotalSettlementIntervalMeasuredDemandControlAreaQty_Ex1uT’I’M’W’VL’mdhcif, except that for a net-settled MSS the UDCTotalSettlementIntervalGrossMeasuredDemandControlAreaQty_Ex1uT’I’M’W’VL’mdhcif output does not net MSS Generation with MSS Demand. |
| 62 | BAUDCTotalSettlementIntervalGrossMeasuredDemandControlAreaQty_Ex1 BuT’I’M’W’VL’mdhcif | UDC Total Gross Measured Demand quantity calculated for a Business Associate and its UDC/MSS over the CAISO Control Area for a settlement interval, with exclusion of Energy provided by Business Associates and resources that comprise Exceptions #1. The output is similar to output BAUDCSettlementIntervalMeasuredDemandControlAreaQty_Ex1BuT’I’M’W’VL’mdhcif except that for a net-settled MSS the BAUDCTotalSettlementIntervalGrossMeasuredDemandControlAreaQty_Ex1uT’I’M’W’VL’mdhcif output does not net MSS Generation with MSS Demand. |
| 63 | BASettlementIntervalNetSettledMSSTotalGrossMeasuredDemandQty_Ex1_MDOverCA BuT’I’M’W’VL’mdhcif | MSS Measured Demand attributable to a Business Associate where the quantity is calculated over all net-settled MSS for a settlement interval, with the exclusion of Energy provided by Business Associates and resources that comprise Exceptions #1. This output is similar to the output BASettlementIntervalUDCTotalNetMSSMeasuredDemandQty_Ex1_MDOverCABuT’I’M’W’VL’mdhcif except that the IntervalNetSettledMSSTotalGrossMeasuredDemandQty_Ex1_MDOverCABuT’I’M’W’VL’mdhcif output does not net MSS Generation with MSS Demand. |
| 64 | BASettlementIntervalNetSettledMSSTotalMeteredCAISODemandQuantity_Ex1_MDOverCA BuT’I’M’W’VL’mdhcif | Sum of meter input values for all Loads and other non-export Demand resources resources that were scheduled by a Business Associate, where the quantity is calculated for all net-settled MSS entities for a settlement interval, with the exclusion of Energy provided by Business Associates and resources that comprise Exceptions #1. The summed quantity is restricted to Energy from net-settled MSS entities. This output is similar to the output BASettlementIntervalUDCTotalMeteredCAISODemandQuantity_Ex1_MDOverCABuT’I’M’W’VL’mdhcif for UDC and gross-settled MSS entities. |
| 65 | BASettlementIntervalNetSettledMSSExportQuantity_Ex1_MDOverCA BuT’I’M’W’VL’mdhcif | Sum of real-time deemed-delivered export quantities, including export losses settled under contractual operating agreements, calculated for a Business Associate where the quantity is calculated over all net-settled MSS entities for a settlement interval, with the exclusion of Energy provided by Business Associates and resources that comprise Exceptions #1. The summed quantity is restricted to Energy from net-settled MSS entities. This output is similar to the output IntervalUDCExportQuantity_Ex1_MDOverCABuT’I’M’W’VL’mdhcif for UDC and gross-settled MSS entities. |
| 66 | CAISOTotalDailyMeasuredDemandControlAreaQty_FERC md | Daily total Measured Demand quantity calculated over the CAISO Control Area, with the exclusion of Energy provided by Business Associates and resources that comprise Exceptions #2 and inclusion of generation for Business Associates and generating units that comprise FERC Fee Exceptions #3. |
| 67 | CAISOTotalSettlementIntervalMeasuredDemandControlAreaQty_FERC mdhcif | Total Measured Demand quantity calculated over the CAISO Control Area for a settlement interval, with the exclusion of Energy provided by Business Associates and resources that comprise Exceptions #2 and inclusion of generation for Business Associates and generating units that comprise FERC Fee Exceptions #3. |
| 68 | BADailyMeasuredDemandControlAreaQty_FERC Bmd | Daily Measured Demand quantity calculated for a Business Associate over the CAISO Control Area, with the exclusion of Energy provided by Business Associates and resources that comprise Exceptions #2 and the inclusion of generation for Business Associates and generating units that comprise FERC Fee Exceptions #3. |
| 69 | BASettlementIntervalMeasuredDemandControlAreaQty_FERC Bmdhcif | Measured Demand quantity calculated for a Business Associate over the CAISO Control for a settlement interval, with exclusion of Energy provided by Business Associates and resources that comprise Exceptions #2 and inclusion of generation for Business Associates and generating units that comprise FERC Fee Exceptions #3. |
| 70 | BASettlementIntervalTotalMeteredCAISODemandQuantity_FERC Bmdhcif | Sum of meter input values for all Loads and other non-export Demand resources resources r that were scheduled by Business Associate for a settlement interval, with exclusion of Energy provided by Business Associates and resources that comprise Exceptions #2. The summed quantity is restricted to Energy from non-MSS entities or from gross-settled MSS entities. |
| 71 | BASettlementIntervalTotalNetMSSMeasuredDemandQty_FERC Bmdhcif | Sum of net MSS Measured Demand attributable to a Business Associate for a settlement interval, with exclusion of Energy provided by Business Associates and resources that comprise Exceptions #2. |
| 72 | BASettlementIntervalExportQuantity_FERC Bmdhcif | Sum of Real-Time Deemed-Delivered export quantities plus export losses settled under contractual operating agreements, calculated for a Business Associated and settlement interval, with exclusion of input values for Business Associates and resources comprising Exceptions #2. The calculated quantity contains only export Energy from non-MSS entities and from gross-settled MSS entities. |
| 73 | BASettlementIntervalGenerationQuantity_FERC Bmdhcif | Sum of meter input values for all generating unit resources that were scheduled by a Business Associate for a settlement interval, with inclusion of generation for Business Associates and generating units that comprise FERC Fee Exceptions #3. The summed quantity is restricted to Energy from non-MSS entities or from gross-settled MSS entities. |
| 74 | CAISOHourlyDAGrossMeasuredDemand mdh | Hourly Measured Demand over the CAISO Control Area, independent of the MSS Gross/Net settlement option, where Gross Load is used for the metered CAISO Demand of a net-settled MSS entity. |
| 75 | BAResIntervalUDCMeteredCAISODemandQuantity BrtuT’I’Q’M’AA’R’pPW’Qd’Nz’VvHn’L’mdhcif | CAISO metered Demand that is attributable to Load or other non-export Demand resource for a settlement interval, where T’ = ‘UDC’ (indicating a UDC entity). |
| 76 | BAResIntervalGrossMSSMeteredCAISODemandQuantity BrtuT’I’Q’M’AA’R’pPW’Qd’Nz’VvHn’L’mdhcif | CAISO metered Demand that is attributable to Load or other non-export Demand resource for a settlement interval, where T’ = ‘MSS’ and I’ = ‘GROSS’ (indicating a gross-settled MSS entity). |
| 77 | BAResIntervalNetMSSMeteredCAISODemandQuantity BrtuT’I’Q’M’AA’R’pPW’Qd’Nz’VvHn’L’mdhcif | CAISO metered Demand that is attributable to Load or other non-export Demand resource for a settlement interval, where S’ = ‘GL’ (indicating a MSS gross Load resource) and both T’ = ‘MSS’ and I’ = ‘NET’ (indicating a net-settled MSS entity). |
| 78 | BAHourlyResMeteredDemandControlAreaQty_BCR BrtuT’I’M’F’W’S’VL’mdh | Hourly Gross Metered Demand quantity calculated for a Business Associate over the CAISO Control Area. |
| 79 | BAHourlyResMeteredDemandMinusTORControlAreaQty_BCR BrtuT’I’M’F’W’S’VL’mdh | Hourly total Metered Demand quantity minus Demand served through TOR transmission rights of a Business Associate.This output excludes Energy provided by Business Associates and resources that comprise Exceptions #6 (BCR Exceptions). |
| 80 | BASettlementIntervalResMeteredDemandMinusTORControlAreaQty_BCR BrtuT’I’M’F’W’S’VL’mdhcif | Total Metered Demand quantity minus Demand served through TOR transmission rights of a Business Associate. , This output applies to a settlement interval and excludes Energy provided by Business Associates and resources that comprise Exceptions #6 (BCR Exceptions). |
| 81 | BASettlementIntervalResTOR_MDControlAreaQty_BCR BrtF’S’mdhcif | Total TOR transmission rights quantity that is exempt from inclusion in the Metered Demand outputs for a Business Associate. This output applies to a settlement interval. |
| 82 | BASettlementIntervalResMeteredDemandControlAreaQty_BCR BrtuT’I’M’F’W’S’VL’mdhcif | Gross Metered Demand quantity calculated for a Business Associate over the CAISO Control Area for a settlement interval. |
| 83 | BAResSettlementIntervalUDCMeteredCAISODemandQuantity BrtuT’I’Q’M’AA’F’R’pPW’QS’d’Nz’VvHn’L’mdhcif | Metered Demand quantity calculated for a Business Associate over the CAISO Control Area for a settlement interval, where T’ = ‘UDC’ (indicating a UDC entity). |
| 84 | BAResSettlementIntervalGrossMSSMeteredCAISODemandQuantity BrtuT’I’Q’M’AA’F’R’pPW’QS’d’Nz’VvHn’L’mdhcif | Metered Demand quantity calculated for a Business Associate over the CAISO Control Area for a settlement interval, where T’ = ‘MSS’ and I’ = ‘GROSS’ (indicating a gross-settled MSS entity). |
| 85 | BAResSettlementIntervalNetMSSMeteredCAISODemandQuantity BrtuT’I’Q’M’AA’F’R’pPW’QS’d’Nz’VvHn’L’mdhcif | Gross Metered Demand quantity calculated for a Business Associate over the CAISO Control Area for a settlement interval, where S’ = ‘GL’ (indicating a MSS gross Load resource) and both T’ = ‘MSS’ and I’ = ‘NET’ (indicating a net-settled MSS entity). |
| 86 | BAHourlyResRealTimeTORLoadQty_Ex6_BCR Brmdh | Hourly total Real-Time TOR transmission rights quantity (utilized by CC 6806 RUC Tier1 Allocation) for a Load Resource and Business Associate. This output excludes Energy provided by Business Associates and resources that comprise Exceptions #6 (BCR Exceptions). |
| 87 | BAHourlyResDayAheadTORLoadQty_Ex6_BCR Brmdh | Hourly total Day-Ahead TOR transmission rights quantity (utilized by CC 6806 RUC Tier1 Allocation) for a Load Resource and Business Associate. This output excludes Energy provided by Business Associates and resources that comprise Exceptions #6 (BCR Exceptions). |
| 88 | CAISOTotalHourlyMeasuredDemandMinusRightsControlAreaQty_LFEx6 mdh | Hourly total Measured Demand quantity minus Demand served through transmission rights of a Business Associates comprising Exceptions #5, where the Measured Demand quantity represents the Demand of the CAISO Control Area and excludes Energy provided by Business Associates and resources that comprise Exceptions #6. The output reflects the net settlement of a Load-Following MSS regardless of whether or not the MSS has selected the MSS Load-Following option. |
| 89 | CAISOTotalSettlementIntervalMeasuredDemandMinusRightsControlAreaQty_LFEx6 mdhcif | Total Measured Demand quantity minus Demand served through transmission rights of Business Associates comprising Exceptions #5, where the Measured Demand quantity represents the Demand of the CAISO Control Area for a settlement interval and excludes Energy provided by Business Associates and resources that comprise Exceptions #6. The output reflects the net settlement of a Load-Following MSS regardless of whether or not the MSS has selected the MSS Load-Following option. |
| 90 | BAHourlyMeasuredDemandMinusRightsControlAreaQty_LFEx6 Bmdh | Measured Demand quantity minus Demand served through transmission rights of a Business Associate who belongs to Exceptions #5, where the Measured Demand quantity represents the Demand served by the Business Associate over the CAISO Control Area and excludes Energy provided by Business Associates and resources that comprise Exceptions #6. The output reflects the net settlement of a Load-Following MSS regardless of whether or not the MSS has selected the MSS Load-Following option. |
| 91 | BASettlementIntervalMeasuredDemandMinusRightsControlAreaQty_LFEx6 Bmdhcif | Measured Demand quantity minus Demand served through transmission rights of a Business Associate who belongs to Exceptions #5, where the Measured Demand quantity represents the Demand served by the Business Associate over the CAISO Control Area for a settlement interval and excludes Energy provided by Business Associates and resources that comprise Exceptions #6. The output reflects the net settlement of a Load-Following MSS regardless of whether or not the MSS has selected the MSS Load-Following option. |
| 92 | BASettlementIntervalTOR_MDControlAreaQty_BCR Bmdhcif | Total TOR transmission rights quantity that is exempt from inclusion in the Metered Demand outputs for a Business Associate. This output applies to a settlement interval. |
| 93 | CAISOTotalHourlyMeasuredDemandControlAreaQty_LFEx6 mdh | Hourly total Measured Demand quantity calculated over the CAISO Control Area, with the exclusion of Energy provided by Business Associates and resources that comprise Exceptions #6. The output reflects the net settlement of a Load-Following MSS regardless of whether or not the MSS has selected the MSS net-settlement option. |
| 94 | CAISOTotalSettlementIntervalMeasuredDemandControlAreaQty_LFEx6 mdhcif | Total Measured Demand quantity calculated over the CAISO Control Area for a settlement interval, with the exclusion of Energy provided by Business Associates and resources that comprise Exceptions #6. The output reflects the net settlement of a Load-Following MSS regardless of whether or not the MSS has selected the MSS Load-Following option. |
| 95 | BAHourlyMeasuredDemandControlAreaQty_LFEx6 Bmdh | Hourly Measured Demand quantity calculated for a Business Associate over the CAISO Control Area with exclusion of Energy provided by Business Associates and resources that comprise Exceptions #6. The output reflects the net settlement of a Load-Following MSS regardless of whether or not the MSS has selected the MSS Load-Following option. |
| 96 | BASettlementIntervalMeasuredDemandControlAreaQty_LFEx6 Bmdhcif | Measured Demand quantity calculated for a Business Associate over the CAISO Control Area for a settlement interval, with the exclusion of Energy provided by Business Associates and resources that comprise Exceptions #6. The output reflects the net settlement of a Load-Following MSS regardless of whether or not the MSS has selected the MSS Load-Following option. |
| 97 | UDCTotalSettlementIntervalMeasuredDemandControlAreaQty_LFEx6 uT’I’M’W’VL’mdhcif | UDCTotal Measured Demand quantity calculated over the CAISO Control Area for a UDC/MSS for a settlement interval, with the exclusion of Energy provided by Business Associates and resources that comprise Exceptions #6. The output reflects the net settlement of a Load-Following MSS regardless of whether or not the MSS has selected the MSS Load-Following option. |
| 98 | BAUDCHourlyMeasuredDemandControlAreaQty_LFEx6 BuT’I’M’W’VL’mdh | Hourly UDC Measured Demand quantity calculated for a Business Associateand its UDC/MSS over the CAISO Control Area, with exclusion of Energy provided by Business Associates and resources that comprise Exceptions #6. The output reflects the net settlement of a Load-Following MSS regardless of whether or not the MSS has selected the MSS Load-Following option. |
| 99 | BAUDCSettlementIntervalMeasuredDemandControlAreaQty_LFEx6 BuT’I’M’W’VL’mdhcif | UDC Measured Demand quantity calculated for a Business Associate and its and UDC/MSS over the CAISO Control Area for a settlement interval, with exclusion of Energy provided by Business Associates and resources that comprise Exceptions #6. The output reflects the net settlement of a Load-Following MSS regardless of whether or not the MSS has selected the MSS Load-Following option. |
| 100 | BASettlementIntervalUDCTotalMeteredCAISODemandQuantity_LFEx6_MDOverCA BuT’I’M’W’VL’mdhcif | Sum of meter input values for all Load and other non-export Demand resources that were scheduled by a Business Associate, where the quantity is calculated for all UDC and gross-settled MSS entities for a settlement interval, with exclusion of Energy provided by Business Associates and resources that comprise Exceptions #6. The summed quantity is restricted to Energy from non-MSS entities or from gross-settled MSS entities. The output reflects the net settlement of a Load-Following MSS regardless of whether or not the MSS has selected the MSS Load-Following option. |
| 101 | BASettlementIntervalUDCTotalNetMSSMeasuredDemandQty_LFEx6_MDOverCA BuT’I’M’W’VL’mdhcif | Net MSS Measured Demand attributable to a Business Associate where the quantity is calculated in association with a net-settled MSS entity for a settlement interval, with exclusion of Energy provided by Business Associates and resources that comprise Exceptions #6. The output reflects the net settlement of a Load-Following MSS regardless of whether or not the MSS has selected the MSS Load-Following option. |
| 102 | BASettlementIntervalUDCExportQuantity_Ex6_MDOverCA BuT’I’M’W’VL’mdhcif | Sum of real-time deemed-delivered export quantities, including export losses settled under contractual operating agreements, calculated for a Business Associate for a settlement interval, with exclusion of Energy provided by Business Associates and resources that comprise Exceptions #6. |
| 103 | BAHourlyMeasuredDemandMinusRightsQuantity_NON_LF_EX_RTM_BCR Bmdh | Hourly BCR Tier 1 Measured Demand quantity minus Demand served through transmission rights of a Business Associate who belongs to Exceptions #5, where the Measured Demand quantity represents the Demand served by the Business Associate over the CAISO Control Area and excludes Energy provided by Business Associates and resources that comprise Exceptions #6. The output reflects the net settlement of a Load-Following MSS has not selected the MSS Load-Following option (where L’ = ‘NO’). For a Load-Following MSS (where L’ = ‘YES’), the Measured Demand quantity = 0. |
| 104 | BAHourlyTORContractDemandQuantity_EX_BCR Bmdh | Hourly Total TOR transmission rights quantity that is exempt from inclusion in the Metered Demand outputs for a Business Associate. |
| 105 | BAHourlyUDCOrMSSMeasuredDemandQuantity_NON_LF_EX_RTM_BCR Bmdh | Hourly BCR Tier 1 Measured Demand quantity, where the Measured Demand quantity represents the Demand served by a Business Associate over the CAISO Control Area and excludes Energy provided by Business Associates and resources that comprise Exceptions #6. The output reflects the net settlement of a Load-Following MSS has not selected the MSS Load-Following option (where L’ = ‘NO’). For a Load-Following MSS (where L’ = ‘YES’), the Measured Demand quantity = 0. |
| 106 | BASettlementIntervalMSSNetMeasuredDemandQuantity_NON_LF_EX_RTM_BCR BuT’I’M’W’VL’mdhcif | Net MSS Measured Demand attributable to a Business Associate, where the quantity is calculated in association with a net-settled MSS entity for a settlement interval, with exclusion of Energy provided by Business Associates and resources that comprise Exceptions #6. The output reflects the net settlement of a Load-Following MSS has not selected the MSS Load-Following option (where L’ = ‘NO’). For a Load-Following MSS (where L’ = ‘YES’), the net Measured Demand quantity = 0. |
| 107 | CAISOHourlyMeasuredDemandMinusBalancedRightsQuantity_EX_RTM_CONGOFF mdh | Hourly total Measured Demand quantity (in MWh) calculated over the CAISO Control Area, with the exclusion of Energy provided by Business Associates and resources belonging to Exception Set #7 and the exclusion of Measured Demand associated with contract rights. The output reflects the net settlement of a MSS entity regardless of whether or not the MSS has selected the MSS net-settlement option. |
| 108 | BAHourlyMeasuredDemandMinusBalancedRightsQuantity_EX_RTM_CONGOFF Bmdh | Hourly Measured Demand quantity (in MWh) associated with a Business Associate , with the exclusion of Energy provided by Business Associates and resources belonging to Exception Set #7, and the exclusion of Measured Demand associated with contract rights. The output reflects the net settlement of a MSS entity regardless of whether or not the MSS has selected the MSS net-settlement option. |
| 109 | BASettlementIntervalMeasuredDemandMinusBalancedRightsQuantity_EX_RTM_CONGOFF Bmdhcif | Measured Demand quantity (in MWh) associated with Business Associate for a settlement interval, with the exclusion of Energy provided by Business Associates and resources belonging to Exception Set #7, and the exclusion of Measured Demand associated with contract rights. The output reflects the net settlement of a MSS entity regardless of whether or not the MSS has selected the MSS net-settlement option. |
| 110 | BASettlementIntervalEntityMeasuredDemandMinusBalancedRightsQuantity_EX_RTM_CONGOFF BuT’I’M’W’VL’mdhcif | Measured Demand quantity (in MWh) for a Business Associate, for a settlement interval with the exclusion of Energy provided by resources belonging to Exception Set #7, and the exclusion of Measured Demand associated with contract rights. |
| 111 | BASettlementIntervalMSSNetMeasuredDemandMinusBalancedRightsQuantity_EX_CONGOFF BuT’I’M’W’VL’mdhcif | Net MSS Measured Demand quantity (in MWh) for a Business Associate for a settlement interval, with the exclusion of Energy provided by resources belonging to Exception Set #7, and the exclusion of Measured Demand associated with contract rights. The quantity is intended to be used for MSS cases where T’ = ‘MSS’. |
| 112 | BASettlementIntervalNetMSSDemandMinusBalancedRightsNonExportTotalQuantity_EX_CONGOFF BuT’I’M’W’VL’mdhcif | Net MSS Demand quantity (in MWh) for a Business Associate, for a settlement interval, with the exclusion of Energy provided by resources belonging to Exception Set #7, and the exclusion of Measured Demand associated with contract rights. The quantity is intended to be used for MSS cases where T’ = ‘MSS’. |
| 113 | BASettlementIntervalNetMSSDemandTotalQuantity BuT’I’M’W’VL’mdhcif | Net MSS Demand quantity (in MWh) for a Business Associate for a settlement interval. The quantity is intended to be used for MSS cases where T’ = ‘MSS’. |
| 114 | BASettlementIntervalEntityExceptionsNonExportTotalQuantity_EX_CONGOFF BuT’I’M’W’VL’mdhcif | Non-Export resource exceptions Energy quantity (in MWh) for a Business Associate, for a settlement interval, where the Energy represents metered CAISO Demand that is attributable to resources that belong to Exception Set #7. |
| 115 | BASettlementIntervalEntityExceptionsQuantity_EX_CONGOFF BtuT’I’M’AA’W’VL’mdhcif | Exceptions Energy quantity (in MWh) from Demand resources of a Business Associate for a settlement interval, where the Energy is attributable to resources that belong to Exception Set #7. |
| 116 | SettlementIntervalDeemedDeliveredInterchangeEnergyExportTotalQuantity BrtuT’I’Q’M’AA’R’pPW’Qd’Nz’VvHn’L’mdhcif | Deemed-delivered Real-Time Interchange export schedule Energy quantity (in MWh) for a Business Associate, for a settlement interval, where the Energy is attributable to Real-Time Interchance export schedules. |
| 117 | BASettlementIntervalEntityBalancedRightsNonExportTotalQuantity_EX_CONGOFF BuT’I’M’W’VL’mdhcif | Balanced contract rights quantity (in MWh) for Non-Export Resource Demand of a Business Associate for a settlement interval, where the Energy excludes metered CAISO Demand that is attributable to resources that belong to Exception Set #7. |
| 118 | BASettlementIntervalEntityBalancedContractTypeDemandQuantity_EX_CONGOFF BtuT’I’M’AA’W’z’VL’mdhcif | Balanced contract rights quantity (in MWh) for a Business Associate for a settlement interval, where the Energy excludes TOR-covered Demand Energy that is attributable to resources that belong to Exception Set #7. |
| 119 | BASettlementIntervalEntityBalancedContractTypeDemandQuantity BtuT’I’M’AA’W’z’VL’mdhcif | Balanced contract rights quantity (in MWh) for a Business Associate for a settlement interval. |
| 120 | BASettlementIntervalResourceContractTypeMDZero BrtuT’I’Q’M’AA’R’pPW’Qd’z’VvHn’L’mdhcif | Zero-valued charge type used to convey MSS-related attribute information. The zero-value output applies to a settlement interval. |
| 121 | BASettlementIntervalEntityBalancedContractTypeExceptionsDemandQuantity_EX_CONGOFF BtuT’I’M’AA’W’z’VL’mdhcif | Balanced contract rights exceptions quantity (in MWh) for a Business Associate for a settlement interval, where the Energy excludes TOR-covered Demand Energy that is attributable to resources that belong to Exception Set #7. |
| 122 | BASettlementIntervalExportsMinusBalancedRightsExportTotalQuantity_EX_CONGOFF BuT’I’M’W’VL’mdhcif | Energy exports minus balanced contract rights quantity (in MWh) for a Business Associate, for a settlement interval, where the calculated Energy quantity excludes Real-Time Interchange export schedules that are attributable to resources that belong to Exception Set #7. |
| 123 | BASettlementIntervalEntityExportQuantity BuT’I’M’AA’W’VL’mdhcif | Energy exports quantity (in MWh) for a Business Associate for a settlement interval. |
| 124 | BASettlementIntervalEntityQuantity BtuT’I’M’AA’W’VL’mdhcif | Metered CAISO Demand plus Real-Time Interchange export schedule Energy quantity (in MWh) for a Business Associate for a settlement interval. |
| 125 | BASettlementIntervalEntityExceptionsExportQuantity_EX_CONGOFF BuT’I’M’AA’W’VL’mdhcif | Exceptions Energy quantity (in MWh) for a Business Associate for a settlement interval, where the calculated Energy quantity consists of only those Real-Time Interchange export schedules that are attributable to resources that belong to Exception Set #7. |
| 126 | BASettlementIntervalEntityBalancedRightsExportQuantity_EX_CONGOFF BuT’I’M’AA’W’VL’mdhcif | Balanced contract rights quantity (in MWh) for a Business Associate for a settlement interval, where the calculated Energy excludes contract rights of Real-Time Interchange export schedules of system resources that belong to Exception Set #7. |
| 127 | BASettlementIntervalUDCTotalMeteredCAISODemandQuantity_EX_CONGOFF BuT’I’M’W’VL’mdhcif | UDC Metered CAISO Demand quantity (in MWh) for a Business Associate for a settlement interval, where the Entity Type T’ = ‘UDC’ and the Energy excludes resources that belong to Exception Set #7. |
| 128 | BASettlementIntervalEntityNonExportTotalDemandQuantity BuT’I’M’W’VL’mdhcif | Metered CAISO Demand Energy quantity (in MWh) for a Business Associate for a settlement interval. The output includes only the Demand associated with non-Export resources. |
| 129 | BASettlementIntervalUDCExportQuantity_EX_CONGOFF BuT’I’M’W’VL’mdhcif | UDC Real-Time Interchange export schedule Energy quantity (in MWh) for a Business Associate for a settlement interval, where the Entity Type T’ = ‘UDC’, and the output quantity excludes Energy from resources that belong to Exception Set #7. |
| 130 | BASettlementIntervalNonNetBalancedRightsDemandQuantity_EX_CONGOFF BuT’I’M’W’VL’mdhcif | UDC (non-netted) Balanced contract rights quantity (in MWh) for a Business Associate for a settlement interval, where the Entity Type T’ = ‘UDC’ and the calculated output quantity is attributable to the Energy of resources that belong to Exception Set #7. |
| 131 | BASettlementIntervalEntityBalancedRightsExportTotalQuantity_EX_CONGOFF BuT’I’M’W’VL’mdhcif | Balanced contract rights quantity (in MWh) for a Business for a settlement interval, where the calculated Energy quantity excludes contract rights of Real-Time Interchange export schedules of system resources that belong to Exception Set #7. |
| 132 | CAISOSettlementIntervalMeasuredDemandMinusBalancedTORLossQuantity_EX_RTM_IMBOFF mdhcif | Total Measured Demand quantity (in MWh) calculated over the CAISO Control Area for a settlement interval, with the exclusion of Energy provided by Business Associates and resources belonging to Exception Set #8 and the exclusion of TOR Loss Energy associated with contract rights. The output reflects net settlement for a Load-Following MSS regardless of whether or not the MSS has selected the MSS net-settlement option. |
| 133 | BASettlementIntervalMeasuredDemandMinusBalancedTORLossQuantity_EX_RTM_IMBOFF Bmdhcif | Measured Demand quantity (in MWh) associated with a Business Associate for a settlement interval, with exclusion of Energy provided by a Business Associates and resources belonging to Exception Set #8, and the exclusion of of TOR Loss Energy associated with contract rights. The output reflects the net settlement of a MSS entity regardless of whether or not the MSS has selected the MSS net-settlement option. |
| 134 | BASettlementIntervalEntityMeasuredDemandMinusBalancedTORLossQuantity_EX_RTM_IMBOFF BuT’I’M’W’VL’mdhcif | Measured Demand quantity (in MWh) for a Business Associate for a settlement interval, with the exclusion of Energy provided by resources belonging to Exception Set #8, and the exclusion of of TOR Loss Energy associated with contract rights. |
| 135 | BASettlementIntervalMSSNetMeasuredDemandMinusBalancedTORLossQuantity_EX_IMBOFF BuT’I’M’W’VL’mdhcif | Net MSS Measured Demand quantity (in MWh) for a Business Associate for a settlement interval, with the exclusion of Energy provided by resources belonging to Exception Set #8, and the exclusion of TOR Loss Energy associated with contract rights. The quantity is needed for the Measured Demand calculations in MSS cases where T’ = ‘MSS’ And (I’ = ‘NET’ Or L’ = ‘YES’). |
| 136 | BASettlementIntervalNetMSSDemandMinusBalancedTORNonExportLossTotalQuantity_EX_IMBOFF BuT’I’M’W’VL’mdhcif | Net MSS Demand quantity (in MWh) for a Business Associate for a settlement interval, with exclusion of Energy provided by resources belonging to Exception Set #8, and the exclusion of Energy for which there are balanced TOR contract rights that protect against transmission loss charges. The quantity is needed for the Measured Demand calculations in MSS cases where T’ = ‘MSS’ And (I’ = ‘NET’ Or L’ = ‘YES’). |
| 137 | BASettlementIntervalEntityExceptionsNonExportTotalQuantity_EX_IMBOFF BuT’I’M’W’VL’mdhcif | Non-Export exceptions Energy quantity (in MWh) for a Business Associate for a settlement interval, where the Energy represents metered CAISO Demand that is attributable to resources that belong to Exception Set #8. |
| 138 | BASettlementIntervalEntityExceptionsQuantity_EX_IMBOFF BtuT’I’M’AA’W’VL’mdhcif | Exceptions Energy quantity (in MWh) from Demand resources scheduled by a Business Associate for a settlement interval, where the Energy is attributable to resources that belong to Exception Set #8. |
| 139 | BASettlementIntervalEntityBalancedTORNonExportLossTotalQuantity_EX_IMBOFF BuT’I’M’W’VL’mdhcif | Balanced TOR-covered Loss Energy quantity (in MWh) for Load and other non-Export resources of a Business Associate for a settlement interval, where the Energy has balanced TOR contract rights that protect against transmission loss charges. |
| 140 | BASettlementIntervalEntityBalancedContractTypeDemandLossQuantity_EX_IMBOFF BtuT’I’M’AA’W’z’VL’mdhcif | Balanced contract-covered Loss Energy quantity (in MWh) for a Business Associate for a settlement interval, where the Energy is protected from transmission congestion by contracts of contract type z’ and excludes the Demand of resources that belong to Exception Set #8. |
| 141 | BASettlementIntervalEntityBalancedContractTypeDemandLossQuantity BtuT’I’M’AA’W’z’VL’mdhcif | Balanced contract-protected Loss Energy quantity (in MWh) for a Business Associate for a settlement interval, where the Energy is protected from transmission congestion by contracts of contract type z’. |
| 142 | BASettlementIntervalEntityBalancedContractTypeExceptionsDemandLossQuantity_EX_IMBOFF BtuT’I’M’AA’W’z’VL’mdhcif | Balanced contract-protected Loss Energy exceptions quantity (in MWh) for a Business Associate for a settlement interval, where the Energy is protected from transmission congestion by contracts of contract type z’ and relates to Demand-type resources that belong to Exception Set #8. |
| 143 | BASettlementIntervalExportsMinusBalancedTORExportLossTotalQuantity_EX_IMBOFF BuT’I’M’W’VL’mdhcif | Energy exports minus balanced TOR-covered Energy Export loss quantity (in MWh) for a Business Associate for a settlement interval, where the calculated Energy quantity excludes the Demand and associated contract rights of resources that belong to Exception Set #8. |
| 144 | BASettlementIntervalEntityExceptionsExportQuantity_EX_IMBOFF BuT’I’M’AA’W’VL’mdhcif | Export exceptions Energy quantity (in MWh) for a Business Associate for a settlement interval, where the calculated Energy quantity is associated with only those Real-Time Interchange export schedules that are attributable to resources that belong to Exception Set #8. |
| 145 | BASettlementIntervalEntityBalancedTORExportLossQuantity_EX_IMBOFF BuT’I’M’AA’W’VL’mdhcif | Balanced TOR-covered Energy Export Loss quantity (in MWh) for a Business Associate for a settlement interval, where the calculated Energy quantity consists exclusively of Energy of TOR-covered Real-Time Interchange export schedules that are not associated with resources belonging to Exception Set #8. |
| 146 | BASettlementIntervalUDCTotalMeteredCAISODemandQuantity_EX_IMBOFF BuT’I’M’W’VL’mdhcif | UDC metered CAISO Demand quantity (in MWh) over all Load or other non-export Demand resources for a Business Associate for a settlement interval, where T’ = ‘UDC’ Or (T’ = ‘MSS’ and I’ = ‘GROSS’ And L’ = ‘NO’) and the output excludes Energy from resources that belong to Exception Set #8. |
| 147 | BASettlementIntervalUDCExportQuantity_EX_IMBOFF BuT’I’M’W’VL’mdhcif | UDC Real-Time Interchange export schedule Energy quantity (in MWh) for a Business Associate for a settlement interval, where T’ = ‘UDC’ Or (T’ = ‘MSS’ and I’ = ‘GROSS’ and L’ = ‘NO’)’, and the output excludes Energy from resources that belong to Exception Set #8. |
| 148 | BASettlementIntervalNonNetBalancedTORDemandLossQuantity_EX_IMBOFF BuT’I’M’W’VL’mdhcif | UDC (non-netted) balanced TOR-covered Loss Energy quantity (in MWh) for a Business Associate for a settlement interval, where T’ = ‘UDC’ Or T’ = ‘MSS’ And I’ = ‘GROSS’ And L’ = ‘NO’, and where the calculated output quantity is comprised of a Load/non-export Demand component and an Energy Export Demand component, the calculated output is attributable to Energy of resources that do not belong to Exception Set #8. |
| 149 | BASettlementIntervalEntityBalancedTORExportLossTotalQuantity_EX_IMBOFF BuT’I’M’W’VL’mdhcif | Balanced TOR-covered Energy Export quantity (in MWh) that is associated with TOR contracts that provide protection for loss quantities. The output applies to a Business Associate for a settlement interval, where the Energy Export resources do not belong to Exception Set #8. |
| 150 | CAISOSettlementIntervalMeasuredDemandMinusBalancedTORDemandQuantity_EX_RTM_IMBOFF mdhcif | Total Measured Demand quantity (in MWh) calculated over the CAISO Control Area for a settlement interval, with the exclusion of Energy provided by Business Associates and resources belonging to Exception Set #8 and the exclusion of TOR Demand Energy associated with contract rights. The output reflects net settlement for a Load-Following MSS regardless of whether or not the MSS has selected the MSS net-settlement option. |
| 151 | BASettlementIntervalMeasuredDemandMinusBalancedTORDemandQuantity_EX_RTM_IMBOFF Bmdhcif | Measured Demand quantity (in MWh) associated with a Business Associate for a settlement interval, with the exclusion of Energy provided by a Business Associate or resources belonging to Exception Set #8, and the exclusion of of TOR Demand Energy associated with contract rights. The output reflects the net settlement of a MSS entity regardless of whether or not the MSS has selected the MSS net-settlement option. |
| 152 | BASettlementIntervalEntityMeasuredDemandMinusBalancedTORDemandQuantity_EX_RTM_IMBOFF BuT’I’M’W’VL’mdhcif | Measured Demand quantity (in MWh) for a Business Associate for a settlement interval, with the exclusion of Energy provided by a BA’s resources belonging to Exception Set #8, and the exclusion of of TOR Demand Energy associated with contract rights. |
| 153 | BASettlementIntervalMSSNetMeasuredDemandMinusBalancedTORDemandQuantity_EX_IMBOFF BuT’I’M’W’VL’mdhcif | Net MSS Measured Demand quantity (in MWh) for a Business Associate for a settlement interval, with the exclusion of Energy provided by resources belonging to Exception Set #8, and the exclusion of TOR Demand Energy associated with contract rights. The quantity is needed for the Measured Demand calculations in MSS cases where T’ = ‘MSS’ And (I’ = ‘NET’ Or L’ = ‘YES’). |
| 154 | BASettlementIntervalNetMSSDemandMinusBalancedTORNonExportDemandTotalQuantity_EX_IMBOFF BuT’I’M’W’VL’mdhcif | Net MSS Demand quantity (in MWh) for a Business Associate for a settlement interval, with the exclusion of Energy provided by resources belonging to Exception Set #8, and the exclusion of TOR Demand Energy associated with contract rights for Load and other non-export Demand resources. The quantity is needed for the Measured Demand calculations in MSS cases where T’ = ‘MSS’ And (I’ = ‘NET’ Or L’ = ‘YES’). |
| 155 | BASettlementIntervalEntityBalancedTORNonExportTotalQuantity_EX_IMBOFF BuT’I’M’W’VL’mdhcif | Balanced TOR-covered Demand Energy quantity (in MWh) not associated with Export resources for a Business Associate for a settlement interval, where the Energy is protected from transmission congestion by contracts and excludes Demand from resources that belong to Exception Set #8. |
| 156 | BASettlementIntervalEntityBalancedContractTypeDemandQuantity_EX_IMBOFF BtuT’I’M’AA’W’z’VL’mdhcif | Balanced, contract-protected Demand Energy quantity (in MWh) for contracts that are associated with a Business Associate, where the Energy is aggregated over Demand-type resources for a settlement interval, and where the Energy excludes Demand from resources that belong to Exception Set #8. |
| 157 | BASettlementIntervalEntityBalancedContractTypeExceptionsDemandQuantity_EX_IMBOFF BtuT’I’M’AA’W’z’VL’mdhcif | Balanced contract-protected Demand Energy exceptions quantity (in MWh) for a Business Associate for a settlement interval, where the Energy is protected from transmission congestion by contracts and excludes Demand from resources that belong to Exception Set #8. |
| 158 | BASettlementIntervalExportsMinusBalancedTORExportTotalQuantity_EX_IMBOFF BuT’I’M’W’VL’mdhcif | Energy exports minus balanced TOR-covered Energy Export quantity (in MWh) for a Business Associate for a settlement interval, where the calculated Energy quantity excludes TOR-covered Demand for Real-Time Interchange export schedules that are attributable to resources belonging to Exception Set #8. |
| 159 | BASettlementIntervalEntityBalancedTORExportQuantity_EX_IMBOFF BuT’I’M’AA’W’VL’mdhcif | Balanced TOR-covered Energy Export quantity (in MWh) for a Business Associateor a settlement interval, where the calculated Energy quantity and consists exclusively of TOR-covered transmission losses for Real-Time Interchange export schedules that are attributable to resources that do not belong to Exception Set #8. |
| 160 | BASettlementIntervalNonNetBalancedTORDemandQuantity_EX_IMBOFF BuT’I’M’W’VL’mdhcif | UDC (non-netted) balanced TOR-covered Demand Energy quantity (in MWh) for a Business Associate for a settlement interval, where T’ = ‘UDC’ Or T’ = ‘MSS’ And I’ = ‘GROSS’ And L’ = ‘NO’, and where the calculated output quantity is attributable to the Energy of Demand-type resources that do not belong to Exception Set #8, and the Energy Export component of the output quantity is summed over Energy Types ‘FIRM’, ‘NFRM’, ‘WHEEL’, ‘DYN’ and ‘UCTG’. |
| 161 | BASettlementIntervalEntityBalancedTORExportTotalQuantity_EX_IMBOFF BuT’I’M’W’VL’mdhcif | Balanced TOR-covered Energy Export quantity (in MWh) for a Business Associate for a settlement interval, where the calculated Energy quantity consists exclusively of Energy of TOR-covered Real-Time Interchange export schedules that are not associated with resources belonging to Exception Set #8. |
| 162 | BAResource10MMeteredTACAreaDemandQuantity Brtvmdhi | Metered Demand quantity (in MWh) calculated over a TAC Area for a Business Associate and 10-minute interval i. |
| 163 | BAResource10MTORContractMD Brtmdhi | Metered Demand protected by TOR contract rights (that remain after the balancing of source and sink schedules having contract rights) for Business Associate B, Resource ID r, Resource Type t, for a 10-minute interval. |
| 164 | CAISOTotalHourlyMeasuredDemandMinusBalancedTOR_DeclinedHASPBidsQty mdh | Hourly total Measured Demand without TOR protection (in MWh) for all Business Associates, where the quantity is calculated over the CAISO Control Area The output excludes Energy associated with Business Associates and Resources for which Exception Set #9 applies. |
| 165 | BAHourlyMeasuredDemandMinusBalancedTOR_DeclinedHASPBidsQty Bmdh | Hourly BA-specific Measured Demand without TOR protection (in MWh), where the quantity is calculated over the CAISO Control Area for a Business Associate The output excludes Energy associated with BAs and Resources for which Exception Set #9 applies. |
| 166 | BAHourlyMeasuredDemand_DeclinedHASPBidsQty Bmdh | The BA-specific Measured Demand before the subtraction of balanced TOR contract rights (in MWh), where the quantity is calculated over the CAISO Control Area for a Business Associate. The output excludes Energy associated with BAs and Resources for which Exception Set #9 applies. |
| 167 | BAUDCSettlementIntervalMeasuredDemandControlAreaDeclinedHASPBidsQty BuT’I’M’W’VL’mdhcif | The Measured Demand (in MWh) for a specific Business Associate and UDC combination, where the quantity is calculated over the CAISO Control Area for a settlement interval. The output excludes Energy associated with Business Associates and Resources for which Exception Set #9 applies. |
| 168 | BASettlementIntervalUDCTotalMeteredCAISODemandQuantity_Ex9_MDOverCA BuT’I’M’W’VL’mdhcif | The Metered CAISO Demand (in MWh) for a specific Business Associate and UDC combination for a settlement interval, , where T’ = ‘UDC’ or T’ = ‘MSS’ and I’ = ‘GROSS’. The output excludes Energy associated with BAs and Resources for which Exception Set #9 applies. |
| 169 | BASettlementIntervalUDCTotalNetMSSMeasuredDemandQty_Ex9_MDOverCA BuT’I’M’W’VL’mdhcif | The net Measured Demand (in MWh) for a specific Business Associate and MSS combination for a settlement interval , where T’ = ‘MSS’ and I’ = ‘NET’. The output excludes Energy associated with BAs for which Exception Set #9 applies independently of Resource ID. |
| 170 | BASettlementIntervalUDCExportQuantity_Ex9_MDOverCA BuT’I’M’W’VL’mdhcif | The Real-Time Interchange Export Schedules (in MWh) for a specific Business Associate and UDC combination, where the quantity is calculated over the CAISO Control Area for a settlement interval, where T’ = ‘UDC’ or T’ = ‘MSS’ and I’ = ‘GROSS’. The output excludes Energy associated with Business Associates and Resources for which Exception Set #9 applies. |
| 171 | BAHourlyBalancedTOR_DeclinedHASPBidsQty Bmdh | Hourly TOR transmission rights (in MWh) of a Business Associate. Up to the value of the output quantity, the Business Associate is excluded from Measured Demand. |
| 172 | CAISOTotal10MMeasuredDemandControlAreaQty mdhi | Total Measured Demand quantity calculated over the CAISO Control Area for a 10-minute interval. |
| 173 | BA10MMeasuredDemandControlAreaQty Bmdhi | Measured Demand quantity calculated over the CAISO Control Area in for a 10-minute interval. |

# Charge Code Effective Date

| Charge Code/ Pre-calc Name | Document Version | Effective Start Date | Effective End Date | Version Update Type |
| --- | --- | --- | --- | --- |
| BPM - CG PC Measured Demand Over Control Area | 5.0 | 04/01/09 | 03/31/09 | Documentation Edits Only |
| BPM - CG PC Measured Demand Over Control Area | 5.1 | 04/01/09 | 03/31/09 | Documentation Edits and Configuration Impacted |
| BPM - CG PC Measured Demand Over Control Area | 5.2 | 04/01/09 | 03/31/09 | Documentation Edits and Configuration Impacted |
| BPM - CG PC Measured Demand Over Control Area | 5.3 | 04/01/09 | 03/31/09 | Documentation Edits and Configuration Impacted |
| BPM - CG PC Measured Demand Over Control Area | 5.4 | 04/01/09 | 03/31/09 | Documentation Edits and Configuration Impacted |
| BPM - CG PC Measured Demand Over Control Area | 5.5 | 08/27/09 | 03/31/09 | Documentation Edits and Configuration Impacted |
| BPM - CG PC Measured Demand Over Control Area | 5.6 | 03/05/10 | 03/31/09 | Documentation Edits and Configuration Impacted |
| BPM - CG PC Measured Demand Over Control Area | 5.7 | 04/01/09 | 01/31/11 | Documentation Edits and Configuration Impacted |
| BPM - CG PC Measured Demand Over Control Area | 5.8 | 02/01/11 | 12/12/11 | Documentation Edits and Configuration Impacted |
| BPM - CG PC Measured Demand Over Control Area | 5.8a | 12/13/11 | 11/30/12 | Documentation Edits Only |
| BPM - CG PC Measured Demand Over Control Area | 5.9 | 12/01/12 | 04/30/13 | Documentation Edits and Configuration Impacted |
| Documentation Edits and Configuration Impacted | 5.10 | 05/01/13 | 4/30/14 | Documentation Edits and Configuration Impacted |
| BPM - CG PC Measured Demand Over Control Area | 5.11 | 05/01/14 | 9/30/14 | Documentation Edits and Configuration Impacted |
| BPM - CG PC Measured Demand Over Control Area | 5.12 | 10/01/14 | 10/31/14 | Documentation Edits and Configuration Impacted |
| BPM - CG PC Measured Demand Over Control Area | 5.13 | 11/1/14 | 7/31/18 | Documentation Edits and Configuration Impacted |
| BPM - CG PC Measured Demand Over Control Area | 5.13.5 | 8/1/18 | 12/31/20 | Documentation Edits and Configuration Impacted |